# Column Design

4.1 General:

Columns are structural element used primarily to support axial compressive loads, that coming from slabs that stand over them.

4.2 Introduction:

Practically columns are subjected not to axial loads but also to moment from direct loading or end rotation.

Columns are classified to short columns and long columns. A short column is one in which the ultimate load is governed by the strength of materials and dimension of the cross section, A long column one in which ultimate capacity is influenced by slenderness, which produced additional bending  because of transverse deformation, see Appendix C.

4.3 Types of columns:

Reinforced concrete columns are principally classified into three types according to its section.

Tied columns:

Here in this type the bars are braced or tied at internal closed loops called ties, which are usually square or rectangular columns.

Spiral columns:

This type which the bars warped with a closely spaced spiral of small diameter, usually circular column in which spiral reinforcement is used this kind of columns are used in open area.

Composed column:

This type is rarely used in which rolled sections is used in addition to concrete.

4.4Columns classification:

Short columns:

Strength is related to properties of used material, also short columns are limited by the following equation.

1)  Kl/r≤34-12(𝓜u1/ 𝓜u2) ……..short column.

2)  Kl/r>34-12(𝓜u1/ 𝓜u2) .…… Long column

Long column:

Strength is related to slenderness ratio of the column. And here the last equation can't be applied.

Minimum moment imposed in the column:

All columns are assumed to have minimum moment imposed on them defined by minimum eccentricity.

Emin≥ (1.5+0.03h) in either direction

.h= overall dimension of the column at rights angle to the axis of the bending.

M min=P emin

Clear spacing:

For each column clear spacing between bars should be largest of the (1.5db, 1.5 inch, max. aggregate size).

Cover:

Clear cover 5 cm; for exterior exposure.

Spacing between bars:

Spacing between bars should not be less than largest of the following:

1)1.5db

2)1.5 inches.

3)4/3 max. Aggregate size

Spacing between bars =b/n.

Spacing between stirrups:

Spacing between bars should not be exceeding the least of the following:

1) At least dimension of the column.

2)16db

3)48ds

4.5Column design:

According to (ACI-2002) code, the load on the column can be calculated from the following equation.

Pd=фPn=ф*λ {0.85* 𝒇'c (Ag-As) +As* ℱy}

Where:

Ф: reduction factor.

Ф: 0.75 for tied column.

Ф: 0.65 for spiral column.

Ag: gross area of the cross section.

ℱy: yield strength of steel.

Check buckling:

Pcr=π2 (EI)/ (KL) 2

EI= (0.4Ec x Ig)/ (1+Bd).

Bd=1.4 dead/PU.

Buckling will not occur if………..Pu< ФPcr.

Where: Ф reduction factor=0.75

Check Slenderness ratio:

In order to determine whether the columns is short or long:

1)  Kl/r≤34-12(𝓜u1/ 𝓜u2) ……..short column.

2)  Kl/r>34-12(𝓜u1/ 𝓜u2) ….… Long column

Where:

K: effective length factor.

Lu: unbraced length of the column

r: radius of gyration=√I/A

Mu1& Mu2: columns end moments, "smaller and larger respectively"

𝓜u1/ 𝓜u2: Positive for a single curvature and negative for double curvature

Design:

4.5.1  Column Design of Block A

4.5.1.a   Interior column (circular C20):

Fig(4-1)Column (20) in Block A

This column has circular shape and we transfer it to equivalent square D=1m ≈ h=b=0.88 m

DL=158.28 ton, Pu=251.77 ton

𝓜 1=1.16 ton.m,   2=1.22 ton.m

From sap 2000

Check if it long or short column in both direction:

Fig (4-2) Pu from sap2000

In X-X direction:

ΨB=∑(EI/L)C/∑(EI/L)B

= (0.0491/12)/(0.0759/8.75+0.759/10.03)=0.25

ΨT=∑(EI/L)C/∑(EI/L)B

= (0.0491/12)/ (7.2*10-3 /8.75)=4.97

K=0.75      (braced)

R=0.25

Lu=11.2

Kl/r=0.75*11.2/0.25=33.6 > 34-12*1.16/1.22=22.59

Long column in X-X direction

In Y-Y direction:

ΨB= 5.3

ΨT=0.26

(Braced)

K=0.75

R=0.25

Lu=11.2

Kl/r=0.75*11.2/0.25=33.6 > 34-12*16/1.22=22.59

Long column in Y-Y direction

Design of long column:

Bd=1.2D/Pu

=1.2*158.28/251.77 =0.7544

Ieff=0.4*Ig/ (1+ Bd)

=0.4*0.0491/ (1+0.7544)

=0.01119471 m4   =1119471 cm4

Pcr=π2 (EI)/ (KL) 2

= (π2*151000√240*1119471)/ ((0.75*11.2*100)2*1000)

=3663 ton

Cm=0.6+0.4(𝓜 1/𝓜2)

=0.6+0.4(1.16/1.22) =0.98

δns=Cm/(1-Pu/ фPc)

=0.98/(1-251.77/(0.75*3663))

=1.08

𝓜c= δns*𝓜u

=1.08*1.22 =1.32 ton.m

Most columns are subjected to bending moment in addition to axial force.

𝓜min = emin x Pu

Where:

𝓜min: minimum moment required to design columns for bending.

emin: minimum eccentricity

= 0.015+0.03h ; h: column side dimension in m

Pu: ultimate axial force on column.

𝓜u=1.22 ton.m   from sap

emin=0.015+0.03*1=0.045 m

𝓜min= emin*Pu

=0.045*251.77 =11.33 ton.m

𝓜c < min   so that design for moment should not be performed.

The nominal compressive strength for axially loaded column is:

Take As =0.01Ag

Pd=фPn=ф*λ {0.85* 𝒇'c (Ag-As) +As* ℱy}

251.77*1000=0.75*0.85{0.85*240*((π/4)(100)2– As)+As*4200}

As=78.54 cm2

Check strength:

Pd=0.75*0.85{0.85*240*{(π/4) (100) - 78.54) +4200*78.54}

=1221.5 ton >Pu     ok

As= (16 ф25)

Fig ( 4-3 ) As from sap2000

We note that the area of steel from hand calculation is same as from SAP.

Shear Design:

Fig ( 4-4 ) Shear force from sap2000

Vu=0.0048 ton      ,

Nu (axial load) =240.6 ton   from sap2000

ФVc=0.75*0.53√ 𝒇'c*b*d {1+ Nu/140*Ag}

={0.75*0.53√240*88*83{1+240.6*103/140*7854}}/103

=54.82 ton > 0.0048 ton

No need for shear reinforcement use Min shear reinforcement.

[Av/S]min=max  {3.5*bw/ ℱy} or {0.2* bw√ 𝒇'c/ ℱy}

={3.5*88/4200=0.073} or {0.2*88* √240/4200}

[Av/S]min=0.05

Fig ( 4-5 ) As for shear from sap2000

Pitch (3.5 cm 8.5 cm)      Ф=10 mm

2.5 ≤ clear spacing between spirals ≤ 7.5 cm

4.5.1.b Exterior column (square C5):

Fig(4-6) Column (15) in Block A

Dimension:

b=h=60 cm

Ic=0.0108 m4,

IL=0.0225 m4,

IEb=0.2457 m4

ITb=7.2*10-3 m4

DL=150.12 ton, Pu=212.5 ton

𝓜 1=0.88ton.m,   2=1.57 ton.m

from sap 2000

Fig (4-7) Pu from sap2000

In X-X direction:

ΨB=∑ (EI/L) C/∑ (EI/L) B

= (0.0108/9)/ (7.2*10-3 /4.65+7.2*10-3 /5.2)=0.41

ΨT=∑ (EI/L) C/∑ (EI/L) B

= [(0.0108/9) + (0.0108/6)]/ [(0.0225/4.65) +(0.0225/5.2) + (0.2457/4.65)          +0.2457/5.2)]=0.03

K=0.85     (braced)

r=0.173

Lu=8.35

Kl/r=0.58*8.35/0.173=28< 34+12*0.88/1.57=34.56

Short column in X-X direction

In Y-Y direction:

ΨB= 0.87

ΨT=0.69

(Braced)

K=0.74

R=0.173

Lu=8.35

KL/r=0.74*8.35/0.173=35.72 > 34.65

Long column in Y-Y direction

Design of long column:

Bd  =1.2D/Pu

=1.2*150.12/212.5 =0.848

Ieff =0.4*Ig/ (1+ Bd)

=0.4*0.0108/ (1+0.848)

=2.34*10-3 m4   =233766 cm4

Pcr=π2 (EI)/ (KL) 2

= (π2*151000√240*233766)/ ((0.74*8.35*100)2*1000)

=1413.6 ton

Cm=0.6+0.4(𝓜 1/𝓜2)

=0.6-0.4(0.88/1.57) =0.376

δNs=Cm/ (1-Pu/ фPc)

=0.376/ (1-212.5/ (0.75*1413.6))

=0.47

𝓜c= δns*𝓜u

=0.47*1.57 =0.738 ton.m

𝓜u=1.57ton.m   from sap

emin=0.015+0.03*0.6=0.033 m

𝓜min= emin*Pu

=0.033*212.5 =7.02 ton.m

𝓜c <𝓜 min   so that design for moment should not be performed.

The nominal compressive strength for axially loaded column is:

Take As =0.01Ag

AS=0.01Ag =36cm2

As= (8 ф 25)

Fig ( 4-8 ) As for shear from sap2000

Check strength:

Pd=фPn=ф*λ {0.85* 𝒇'c (Ag-As) +As* ℱy}

=0.75*0.85{0.85*240*0.99*3600+36*4200}

=560 ton > Pu

Shear Design:

Fig ( 4-9 ) shear force from sap2000

Vu=1.57ton      ,

Nu (axial load) =124.6 ton   from sap2000

ФVc={.75*.53√240*60*55{1+124.6*103/140*3600}}/103

=25.35ton > 1.57 ton

No need for shear reinforcement use Min shear reinforcement.

[Av/S]min=max  {3.5*bw/ ℱy} or {0.2* bw√ 𝒇'c/ ℱy}

={3.5*55/4200=0.073} or {0.2*60* √240/4200=.044}

[Av/S]min=0.05

Fig ( 4-10 )As of shear from sap2000

Use 1Ф10/25 cm.   Ф=10 mm

4.5.2. Column Design of Block B

Fig(4-11)column section for Block B

4.5.2.a Exterior column (rectangular C30):

Dimension:

B=30, h=60 cm

Ic=1.35*10-3 m4,

IB1=0.01 m4,

IEB4=0.1372 m4

Pu=158.28 ton

𝓜1=3.05ton.m,2=3.75ton.m

From sap 2000                                    Fig (4-12  )Pu sap2000

In X-X direction:

ΨB=∑(EI/L)C/∑(EI/L)B

= [(1.35*10-3 /4) + (1.35*10-3 /5)] / (0.01/3.18+0.01/6)

= 0.13

ΨT =∑ (EI/L) C/∑ (EI/L) B

= [(1.35*10-3 /5)/ (0.1372/3.18)] =0

K=0.54     (braced)

r=0.087

Lu=4

Kl/r=0.54*4/0.087=24.83< 34+12*3.05/3.75=43.76

Short column in X-X direction

In Y-Y direction:

ΨB= 0.12

ΨT= 0

(Braced)

K=0.54

R=0.087

Lu=4

KL/r=0.54*4.4/0.087=24.83 < 43.76

Short column in Y-Y direction

Design of short column:

Fig ( 4-13 )As from sap2000

𝓜u/Pu =3.75/158.28=0.024 m

𝜌=𝜌min=0.01

Take As =0.01Ag

AS=0.01*1800=18cm2

As= (8 ф 18)

Check strength:

Pd=фPn=ф*λ {0.85* 𝒇'c (Ag-As) +As* ℱy}

158.28*103=0.65*0.80{0.85*240*0.99Ag+0.01*Ag*4200}

Ag=158.28*103/126.9=1247 cm2

We take 60*30 =1800 cm2   ok

emin=0.1*h

=0.1*0.6=0.06 m

emin > e     ok

𝓜min=emin*Pu

=0.06*158.28 =9.5ton.m

𝓜=3.75 ton.m < 𝓜min.

No need for moment reinforcement.

Shear Design:

Vu=5.53ton

Nu (axial load) =21.8 ton   from sap2000

фVc={0.75*0.53√240*30*55{1+21.8*103/140*1800}}/103

=11.04> 5.53 ton.

No need for shear reinforcement

use Min shear reinforcement.

[Av/S]min=max  {3.5*bw/ ℱy} or {0.2* bw√ 𝒇'c/ ℱy}

={3.5*30/4200=0.073} or {0.2*30* √240/4200=.022}

[Av/S]min=0.05

Use 1Ф10/25 cm.

4.5.2.b Interior column (rectangular C56):

Fig (4-14) Pu from sap2000

Dimension:

B=30, h=60 cm

Ic=1.35*10-3 m4

IB3=6.3*10-3 m4

IB02W=6.3*10-3 m4

Pu=85.45ton

DL=48.06 ton.

𝓜 1=2.89ton.m,

𝓜2=3.14 ton.m

From sap 2000

In X-X direction:

ΨB=∑ (EI/L) C/∑ (EI/L) B

= [(1.35*10-3/4) + (1.35*10-3/5)]/ [(6.3*10-3/5.48+6.3*10-3/2.42)]

=0.16

ΨT=∑ (EI/L) C/∑ (EI/L) B

= (1.35*10-3/5)]/[(6.3*10- 3/5.48+ 6.3*10-3/2.42)]

=0.072

K=0.85      (braced)

r=0.087

Lu=4.4

Kl/r=0.58*4.4/0.087=29.33 > 34-12*2.89/3.14=22.96

Long column in X-X direction

In Y-Y direction:

ΨB= .25

ΨT=0.11

(Braced)

K=0.58

R=0.087

Lu=4.4

Kl/r=0.85*4.4/0.087=29.33> 34-12*2.89/3.14=22.96

Long column in Y-Y direction

Design of long column:

Fig (4-15) As from sap2000

Bd=1.2D/Pu

=1.2*48.06/85.45

=0.675

Ieff=0.4*Ig/ (1+ Bd)

=0.4*1.35*10-3/ (1+0.675)

=3.224*10-4 m4   =32240cm4

Pcr=π2 (EI)/ (KL) 2

= (π2*151000√240*32240)/ ((0.85*4.4*100)2*1000)

=1142.92 ton

Cm=0.6+0.4(𝓜 1/𝓜2)

=0.6+0.4(2.89/3.14) =0.97

δns =Cm/(1-Pu/ фPc)

=0.97/ (1-85.45/ (0.75*1142.93))

=1.077

𝓜c= δns*𝓜u

=1.077*3.14 =3.38 ton.m

emin=0.015+0.03*0.3=0.024m

𝓜min= emin *Pu

=0.024*85.45 =2.015 ton.m

𝓜c < 𝓜min   so that design for moment should not be performed.

The nominal compressive strength for axially loaded column is:

Take As =0.01Ag

As=0.01*Ag=0.01*1800=18 cm2

Check strength:

Pd=фPn=ф*λ {0.85* 𝒇'c (Ag-As) +As* ℱy}

=0.65*0.8{0.85*240(60*30-0.01*1800) +1800*0.01*4200

=244071 Kg =244.07 ton> Pu   ok

Shear Design:

Vu=0.0.853 ton      , Nu (axial load) =82.9 ton   from sap2000

ФVc=0.75*0.53√ 𝒇'c*b*d {1+ Nu/140*Ag}

={0.75*0.53√240*55*30{1+82.9*103/140*1800}}/103

=13.5 ton > 0.0048 ton

No need for shear reinforcement use Min shear reinforcement.

[Av/S]min=max  {3.5*bw/ ℱy} or {0.2* bw√ 𝒇'c/ ℱy}

= {3.5*30/4200=0.073}

Or {0.2*30* √240/4200}

[Av/S]min=0.025

Ф=10 mm

Use 1 Ф10/25 cm.