Smart Watch

Prepared by:

Noor Qan’eer.

Yasmeen Omar.

Supervised by:

Dr.Ala'a Addin Al-Masri

Dr.Lu’ai Malhees

Submitted to the Department of Computer Engineering

in partial fulfillment of the requirements

for the degree of Bachelor of

Computer Engineering

Department of Computer Engineering

College of Engineering

AN-Najah National University

December 2016

# Acknowledgments:

We would like to express our gratitude for everyone help us to finish our graduation project .Starting with our supervisors Dr. Ala'a Addin Al Masri and Dr. Lu’ai Malhees who didn't keep any effort in encouraging us to do a great job, providing us with valuable information and advices . Thanks for the continuous support from our families and friends who support us in all our steps .Thanks to those who helped, stand and paid many efforts to make it easy for us to achieve our dreams. Last, but not least , we would like to thank our parents who educated ,guided, encouraged and did their best to let us be successful and special persons and engineers.

# Disclaimer:

This report was written by students at the computer engineering department , faculty of engineering, An-Najah National University. It has not been altered or corrected, other than additional corrections, as a result of assessment and it may contain language as well as content errors. The views expressed in it together with any outcomes and recommendations are solely those of the students. An-Najah National University accepts no responsibility or liability or the consequences of this report being used for a purpose other than the purpose for which it was it commissioned.

# Table of Content :

Acknowledgments:	3

Disclaimer:	4

Table of Content :	5

List of Figures:	7

List of Tables:	8

Abstract:	9

Chapter 1: Introduction:	11

1.1 Overview:	11

1.2 Motivation:	12

Chapter 2: Constrains and Schedule	13

2.1 Constrains	13

2.2 Schedule:	14

Chapter 3: Literature Review:	15

Chapter 4: Work principle:	16

Chapter 5: Methodology:	17

5.1 Planning for design:	17

5.2 Materials:	17

5.3 Tools:	19

5.3.1 Software programs:	19

5.3.2 Hardware tools:	19

5.3.2.1 Arduino Uno :	19

5.3.2.2 H-Bridge L298N Dual Motor Controller (4)	20

5.3.2.3 Wires and Connectors:	22

5.3.2.4 Robot car kit:	23

5.3.2.5  Stepper Motor :	24

5.3.2.6 Touch Screen : (3)	24

5.3.2.7 Arduino USB cable:	26

5.3.2.8  Power Supply:	26

5.3.2.9  LM393 4-Module Relay (5):	26

5.3.2.10 Bluetooth Module HC-06:	27

Chapter 6: Procedure:	28

6.1 Software Design:	28

6.2 Hardware Design:	30

Chapter 7: Future work:	32

Chapter 8: Results:	33

Chapter 9: Issues and Problems:	34

Chapter 10: Conclusion:	35

Chapter 11: References:	36

# List of Figures:

Fig(1) :Arduino Logo	17

Fig (2): Arduino Uno.	18

Fig (3):H-bridge	18

Fig (4): H-bridge pins	20

Fig (5): Wires	21

Fig (6): car frame	21

Fig (7): Stepper Motor	22

Fig (8): TFT Screen	22

Fig (9): Arduino USB Cable	24

Fig (10): 4-relay Module	25

Fig (11): HC-06	25

Fig (12): Home Page.	26

Fig (13): loads Page	27

Fig (14): Robot Page	27

Fig (15): Loads Part Connection	28

Fig (16): Motor Connection	29

Fig (17): Project result	31

# List of Tables:

Table (1): Schedule.	14

Table (2):  Touch screen Pins and there Usage	25

# Abstract:

This project which is a smart watch aims to control our times , remind us with our activities , make our lives more accurate .

The smart watch has many functionalities . first of all, it controls the working time of electrical devices , save the times for the medicine to be taken ,and when the time came, there is a robot will bring the medicine .Also there is a smart to-do list to manage user tasks that need to be done in specific time .

Now days we have many clocks, alarms and other tools used for time . but the idea is to have a complete time system that take care of different sides of our daily life and make things easier for us using different technologies and designing ways .

Chapter One:

The introduction provides background information to understand the hardware project, the objectives and finally the importance of this project.

Chapter Two and Three:

This chapter reflects the constrains, and how the team group was able to overcome these constrains, furthermore the engineering standards that were used in the project also relevant work.

Chapter Four:

It talks about the work principles.

Chapter Five:

It shows the block diagram of project and lists the components and explains the usage of each component.

Chapter Six:

It shows the procedures and the steps of software and hardware implementation.

Chapter Seven:

The future work and extra features can be added to the project.

Chapter Eight:

Results.

Chapter Nine:

Problems we had faced in the project.

Chapter Ten and Eleven:

Conclusion and References.

# Chapter 1: Introduction:

1.1 Overview:

"A watch that control working of many electrical loads , medical robot and multiple to-do list modules " . This could be a short description for this project also it explains the idea behind the project.

The watch represented by a TFT touch screen shielded with arduino .the first page of the screen is the home page that let you move between the other pages that represent the functionality of the watch. The second page shows the id of devices that will control . we select the  devices that will work and the period that they will work in . the third  page of the screen shows the graph of the home that the robot will move in . and the fourth  page of the screen shows the to-do list modules ids and the time to operate everyone.

## 1.2 Motivation:

As technology occupies the world, this project becomes an essential part in the hardware area, and it aims to be more accurate  and regular .

As a developer we hope that the project will be as the customer expect and to achieve the objectives assigned to it and to increase the customers desire to deal with this kind of watches.

Before starting to bring this project to life we have to search for its importance and asking people and many of them expressed their satisfaction with this project.

# Chapter 2: Constrains and Schedule

## 2.1 Constrains

There are different restrictions that limit any project and make the team deal with many problems in the project especially that any mistake happened can destroy the whole project or parts of it e.g.  If the current or the voltage exceeds a certain limit it will burn the electronic piece. So to bring new tools this is considered as time consuming or simply delay.

In this project many problems appeared and they affected the desired table and the exact goal of the project. These restrictions with regard to time, components  and hardware implementation.

- Time constrains: the time needed to complete this project is nearly three month and half. The limitation was how to bring the suitable components in short time to start the implementation quickly, furthermore the touch screen was not found in everyplace in the country so we wait for almost three weeks to get one from the supplier. All these cause some delays in the work procedure and plan.

- Components constrains: as mentioned previously in the time constrains not all the tools were available and not all of them were compatible to each other.

- Hardware implementation: some components needed to redesign and to try several times to make them work together correctly. All of these constrains leads to the most important constrains the time constrains.

- Software constrains: how to find suitable libraries and drivers to make components as touch screen and Arduino microcontroller working correctly.

## 2.2 Schedule:

| Task Name | Duration | Start | End |
| --- | --- | --- | --- |
| Identify the idea and understand the project requirements. | 21 days | 20/8/2016 | 10/9/2016 |
| Prepare components (SW programs, HW tools) and test them. | 30 days | 10/9/2016 | 10/10/2016 |
| Software implementation(coding) and Testing | 51 days | 10/10/2016 | 30/11/2016 |
| Documentation | 7 days | 1/12/2016 | 7/12/2016 |
| Discuss the project | 1 day | 21/12/2016 | 21/12/2016 |

##### Table (1): Schedule.

# Chapter 3: Literature Review:

The first phase was a study of the existing work in the literature in order to familiarize with the topic. After this phase, we increased our awareness of what the topic is, what is done, and how it is done.

The idea of the project wasn't build from scratch and it was inspired from a different clocks and alarms ideas and their working way.

# Chapter 4: Work principle:

How does the project work?

The smart watch has three functionalities . first one controlling loads and in this part The user needs to choose which devices will work in the coming period and the period they will work in ,there is also a feedback to let the user knows if the device still working or there is an external interrupt affects its work .the second functionality medical robot, if the user become sick and he/she need to get medicine , he/she will enter the time the robot will bring the medicine at . Also note that the robot could be used to bring any thing at a specific time cause the frame of this robot is a car and we can put anything above it and it will bring it on the time. The final functionality  of the watch can be used in the same way as the first one , the user will enter the name and the time of the things he/she wants to do and at what time, then the watch will notify him/her when the time came .

# Chapter 5: Methodology:

## 5.1 Planning for design:

Fig (2): Project Plan.

## 5.2 Materials:

What is Arduino? (1)

Arduino is an open-source prototyping platform based on easy-to-use hardware and software. Arduino boards are able to read inputs - light on a sensor, a finger on a button, or a Twitter message - and turn it into an output - activating a motor, turning on an LED, publishing something online. You can tell your board what to do by sending a set of instructions to the microcontroller on the board. To do so you use the Arduino programming language (based on Wiring), and the Arduino Software (IDE), based on Processing. Over the years Arduino has been the brain of thousands of projects, from everyday objects to complex scientific instruments. A worldwide community of makers - students, hobbyists, artists, programmers, and professionals - has gathered around this open-source platform, their contributions have added up to an incredible amount of accessible knowledge that can be of great help to novices and experts alike.

Arduino was born at the Ivrea Interaction Design Institute as an easy tool for fast prototyping, aimed at students without a background in electronics and programming. As soon as it reached a wider community, the Arduino board started changing to adapt to new needs and challenges, differentiating its offer from simple 8-bit boards to products for IoT applications, wearable, 3D printing, and embedded environments. All Arduino boards are completely open-source, empowering users to build them independently and eventually adapt them to their particular needs. The software, too, is open-source, and it is growing through the contributions of users worldwide.

Why Arduino?

Arduino simplifies the process of working with microcontrollers, but it offers some advantage for teachers, students, and interested amateurs over other systems:

- Inexpensive - Arduino boards are relatively inexpensive compared to other microcontroller platforms. The least expensive version of the Arduino module can be assembled by hand, and even the pre-assembled Arduino modules cost less than $50 (200 NIS).

- Cross-platform - The Arduino Software (IDE) runs on Windows, Macintosh OSX, and Linux operating systems. Most microcontroller systems are limited to Windows.

- Simple, clear programming environment - The Arduino Software (IDE) is easy-to-use for beginners, yet flexible enough for advanced users to take advantage of as well. For teachers, it's conveniently based on the processing programming environment, so students learning to program in that environment will be familiar with how the Arduino IDE works.

- Open source and extensible software - The Arduino software is published as open source tools, available for extension by experienced programmers. The language can be expanded through C++ libraries, and people wanting to understand the technical details can make the leap from Arduino to the AVR C programming language on which it's based. Similarly, you can add AVR-C code directly into your Arduino programs if you want to.

- Open source and extensible hardware - The plans of the Arduino boards are published under a Creative Commons license, so experienced circuit designers can make their own version of the module, extending it and improving it. Even relatively inexperienced users can build the breadboard version of the module in order to understand how it works and save money.

## 5.3 Tools:

### 5.3.1 Software programs:

Arduino IDE 1.6.8: Integrated Development Environment (2):

The open-source Arduino Software (IDE) makes it easy to write code and upload it to the board. It runs on Windows, Mac OS X, and Linux. The environment is written in Java and based on Processing and other open-source software.

#### Fig(1) :Arduino Logo

## 5.3.2 Hardware tools:

### 5.3.2.1 Arduino Uno :

The Arduino Uno is a microcontroller board based on the ATmega328 . It has 14 digital input/output pins (of which 6 can be used as PWM outputs), 6 analog inputs, a 16 MHz ceramic resonator, a USB connection, a power jack, an ICSP header, and a reset button. It contains everything needed to support the microcontroller; simply connect it to a computer with a USB cable or power it with a AC-to-DC adapter or battery to get started.

The Uno differs from all preceding boards in that it does not use the FTDI USB-to-serial driver chip. Instead, it features the Atmega16U2 (Atmega8U2 up to version R2) programmed as a USB-to-serial converter.

"Uno" means one in Italian and is named to mark the upcoming release of Arduino 1.0. The Uno and version 1.0 will be the reference versions of Arduino, moving forward. The Uno is the latest in a series of USB Arduino boards, and the reference model for the Arduino platform.

Fig (2): Arduino Uno.

### 5.3.2.2 H-Bridge L298N Dual Motor Controller (4)

Fig (3):H-bridge

Usage:

H-Bridge's are typically used in controlling motors speed and direction, but can be used for other projects such as driving the brightness of certain lighting projects such as high powered LED arrays.

How it works:

An H-Bridge is a circuit that can drive a current in either polarity and be controlled by Pulse Width Modulation (PWM). Pulse Width Modulation is a means in controlling the duration of an electronic pulse. And motors will last much longer and be more reliable if controlled through PWM.

Pins:

- Out 1: Motor A lead out.

- Out 2: Motor A lead out.

- Out 3: Motor B lead out.

- Out 4: Motor B lead out.

- GND: Ground.

- 5volt: 5volt input (Unnecessary if your power source is 7volt-35volt, if the power source is 7volt-35volt then it can act as a 5volt out).

- EnA: Enables PWM signal for Motor A.

- In1: Enable Motor A.

- In2: Enable Motor A.

- In3: Enable Motor B.

- In4: Enable Motor B.

- EnB: Enables PWM signal for Motor B.

Fig (4): H-bridge pins

Specifications:

- Double H bridge Drive Chip: L298N

- Logical voltage: 5V Drive voltage: 5V-35V

- Logical current: 0-36mA Drive current: 2A (MAX single bridge)

- Max power: 25W

- Dimensions: 43 x 43 x 26mm

- Weight: 26g

- Note: Built-in 5v power supply, when the driving voltage is 7v-35v.

### 5.3.2.3 Wires and Connectors:

There are many kinds of connectors but in this project the connectors which were used are male to female and they were used to connect the H-Bridge to Arduino mega pins especially digital pins.

Fig (5): Wires

### 5.3.2.4 Robot car kit:

The robotic car kit named “front driving and steering “ This car consist of two stepper motors that control the accuracy of motion .it has the following properties : constant accelerator ,the rate of error is absolute   not incremental that’s because of using the stepper motor , and The center of rotation is in the point located at the middle of the line that connects the two motors .

Fig (6): car frame

### 5.3.2.5  Stepper Motor :

Fig (7): Stepper Motor

### 5.3.2.6 Touch Screen : (3)

If you are looking for an inexpensive graphical interface for your Arduino, this could be it. 2.4″ Color TFT display has a four wire resistive touch screen, a micro SD card socket, and a convenient Arduino shield footprint.

Fig (8): TFT Screen

Specifications:

This TFT display is big (2.8" diagonal) bright (4 white-LED backlight) and colorful (18-bit 262,000 different shades)! 240x320 pixels with individual pixel control. It has way more resolution than a black and white 128x64 display. As a bonus, this display has a resistive touchscreen attached to it already, so you can detect finger presses anywhere on the screen.

Pins:

| LCD Shield Pin |  | Use |
| --- | --- | --- |
| 3.3V |  | Power |
| 5V |  | Power |
| GND |  | Power |
| LCD_RD |  | LCD Control |
| LCD_WR/ TOUCH_YP |  | LCD Control / Touch Data (Y+) |
| LCD_RS / TOUCH_XM |  | LCD Control / Touch Data (X-) |
| LCD_CS |  | LCD Control |
| LCD_RST |  | LCD Reset |
| LCD_D2 |  | LCD Data |
| LCD_D3 |  | LCD Data |
| LCD_D4 |  | LCD Data |
| LCD_D5 |  | LCD Data |
| LCD_D6 / TOUCH XP |  | LCD Data/ Touch Data (X+) |
| LCD_D7 / TOUCH YM |  | LCD Data / Touch Data (Y-) |
| LCD_D0 |  | LCD Data |
| LCD_D1 |  | LCD Data |
| SD_CS |  | SD Select |
| SD_DI |  | SD Data |
| SD_DO |  | SD Data |
| SD_SCK |  | SD Clock |

Table (2):  Touch screen Pins and there Usage.

### 5.3.2.7 Arduino USB cable:

The USB cable type is A/B which is a standard USB 2.0 cable. Use it to connect Arduino Uno, Arduino Mega 2560, or any board with the USB female A port of your computer. Cable length is approximately 178cm. The cable has many colors as blue, silver and black etc.

Fig (9): Arduino USB Cable

### 5.3.2.8  Power Supply:

In this project many types of power supply were used, electrical converter to feed Loads part , and re-charged butteries for the robot part .

### 5.3.2.9  LM393 4-Module Relay (5):

This relay module allows you to combine the processing power of the Arduino to devices that use higher current and voltage.   It does so by providing four relays that are rated for 7A at either 28VDC or 10A at 125VAC.

Each relay has  a Normally Open (NO) and a Normally Closed (NC) contact.

With these relays you can control:

- Appliances

- Motors

- Lights

- Other Relay

Fig (10): 4-relay Module

### 5.3.2.10 Bluetooth Module HC-06:

Fig (11): HC-06

# Chapter 6: Procedure:

## 6.1 Software Design:

The software used to program the watch to do the three functionalities that mentioned before .

We program the TFT screen which shielded with Arduino using arduino-c programming language to show four pages .

Home Page :The first page to preview the 3 functionalities names :loads control , robot motion, to-do list modules.

The user will touch the page name he/she want to move to .the microcontroller will read the touch index and compare it with the indexes of the three pages and decide the page that should be shown  so that it  redirect the user to that page .

Fig (12): Home Page.

Loads control: It shows the ids for the four devices that the user will control and their status . which  	reflect a feedback about the device if it’s still working a green mark will appear near it , otherwise red color will appear beside . The user will choose the device and set at which time it will work and set the period that it should be on in .the microcontroller will send logic 1 to the pin that connected with the relay that  corresponding  to the device .

Fig (13): loads Page

Robot page : It shows the map of the home ,we deal with  the map as a tree map and the command that the robot will consider in its way as an array .The robot has initial place that it will set in , and the user will check its place in which room he/she set , then the robot will move to the user place to give him his need and return back to its initial place to be ready for any other call.

Fig (14): Robot Page

To-do list page : it shows the ids of modules that will give alarm when the time came to do the corresponding thing . For example , if I want to make a reminder to read a book , and another reminder to turn the cocker off . I will visit the To-do list page and consider reading book is task # 1 then I will set the time for it , when the time came the microcontroller give logic 1 to the corresponding pin that connected with the led 1 and the led will become plinking .And turning off the cooker is task # 2 , so I will set its time and deal with it in the same way as the first one .

## 6.2 Hardware Design:

Part 1: Electrical Loads .

We connected the Arduino with 4-relays module, and the 4-relay modules connected to the electrical loads ,that need to be controlled . as in the following picture .

Fig (15): Loads Part Connection

Part 2: robot .

To control the motion of motor in accurate way , we used a uni-polar  stepper motor . and the car frame we have , needs two stepper motors , each stepper motor needs to be connected with an H-bridge , that’s why we used two H-bridges connected to the two stepper motors . Then we connect the H-bridge with the Arduino , as shown bellow.

Fig (16): Motor Connection

Part 3: To-do List .

to represent a simple to-do list , we used a leds connected with Arduino pins , and when it’s the time to do the task , the corresponding led will become plinking .

Note :

We used in our project 4 Bluetooth modules . Two modules connected with the clock , one with loads part and the other with robot . cause the Bluetooth module we used (HC-06 ) not supporting the Networking and multiple slaves . so we need 2 bluetooth modules for every communication parts ;to send and receive the needed data.

# Chapter 7: Future work:

- For future expansion of the project, several steps can be taken. The first is to improve the robot design and strategy . we are looking forward adding sensors around the robot to allow it to self walking and notice the obstacles on its way and avoid them .

- Add another functionalities for the clock like praying times .

- Add ability to control loads from outside the home .

- Improve the module design .

# Chapter 8: Results:

The watch is able to successfully turn on and off the loads and get the feedback from the arduino about the status of the loads if they still working or stopped cause  of an external interrupts , and tell the robot about the user room , in order to manage the robot to come and bring objects and returned back to its original place .Also the watch was able to control a small to-do list.

This is shown in the following example when we try to draw a path for the car .the path should make the car move forward then turn left then keep moving forward then stop.

1. we have the loads part ready.                       2.we have the robot tidy and works

3. we have the modules part ready

Fig (17): Project result

# Chapter 9: Issues and Problems:

In this project there were some problems as explained next:

- The Bluetooth module that we use doesn’t support multiple slaves communication and replacing the blutooth modules that we already made for the first part with other tool will consume time.

- Solution:

We have to communication types in our project. One between the clock and loads , we use for this communication two bluetooth modules .And the other communication type is between the clock and the robot , we used another two bluetooth modules to support the other communication.

- The second problem was the power the Arduino memory. We need to store the map of the home in Arduino and this needs large memory , may even more the arduino memory .

- Solution:

We stored the map as a tree map like binaries and this consumes onle 5*5 bits , which is a 25 bits . this nearly 3 Bytes only .

# Chapter 10: Conclusion:

The final performance of the project met all of the basic expectations. The watch was Able to make the functionalities required and control the timing for multiple objects .

Some potential direction for additional capabilities is the addition of more functionality for the watch , to be more general and control all our  events.

# Chapter 11: References:

1.  http://www.arduino.org/products/boards/arduino-uno

2. https://www.arduino.cc

3. http://www.smokeandwires.co.nz/blog/a-2-4-tft-touchscreen-shield-for-arduino

3. http://www.instructables.com/id/How-to-Use-24-TFT-LCD-Shield-With-Arduino- Mega

4. http://www.instructables.com/id/Arduino-Modules-L298N-Dual-H-Bridge-Motor-   Control