ABSTRACT

Air pollution is a global problem that afflicts many regions around the world, and it is one of the problems that we are difficult to solve, but it is possible to reduce it and its sources to some extent.

Many people around the world are dying due to air pollution, with an estimated 7 million people dying due to poor air quality

Palestine is one of the countries that have not escaped this global problem, despite its small area and limited capacity, where some studies have found a noticeable pollution on air quality.

City centers are witnessing a noticeable increase in air pollution rates, and perhaps the most prominent thing that affects the air quality in them is the high traffic congestion, as well as the population increase and its increase in movement and activities that negatively affect the air quality

Perhaps the most prominent of these pollutants that affect air quality PM2.5 are very small particles circulating in the air, with a diameter of 2.5 micrometers or less, and have noticeable effects on human health, as they cause heart and respiratory diseases that may lead to death in some cases

This project includes talk about air quality in West Bank cities, namely Jenin, Nablus and Tulkarm, as well as one of the villages that is Anza Village.

In this project, we found PM2.5 values ​​over 24 continuous days, using the AirU sensor, which measures the PM2.5 amount every minute and accurately

After studying and analyzing the results, it became clear to us that the Jenin region is the most polluted area with PM2.5 particles, where the average percentage of these particles during the study period was 40.05 , followed by Nablus at a rate of 32.28 , followed by Tulkarm at a rate of 29.87 , followed by the village of Anza at a rate of 27.0

And after studying the effect of some weather factors and natural factors : wind speed, the effect of humidity, temperature, elevation of place and precipitation

We get the following results about the Wind Speed, there is a direct relationship between it and the pollutant concentration, and the relationship is inverse

As for  relative humidity, there is a direct relationship between it and the pollutant concentration, And a positive relationship

As for the region’s elevation , there is no relationship between it and the concentration of pollutants. Rather, it affects the difference in temperature day and night  during the summer and winter. As a result, temperature has no clear relationship with the high or low concentration of pollutants

As for the effect of rain, it was evident, as the rate of pollution on rainy days was less than on non-rainy days in the three cities

نبذة مختصرة

تلوث الهواء مشكلة عالمية تصيب مناطق كثيرة حول العالم ، وهي من المشاكل التي يصعب حلها ، ولكن من الممكن الحد منها ومصادرها إلى حد ما.

يموت الكثير من الناس حول العالم بسبب تلوث الهواء ، حيث يموت ما يقدر بنحو 7 ملايين شخص بسبب سوء نوعية الهواء

فلسطين من الدول التي لم تفلت من هذه المشكلة العالمية رغم صغر مساحتها وقدرتها المحدودة ، حيث وجدت بعض الدراسات تلوثا ملحوظا في جودة الهواء.

تشهد مراكز المدن زيادة ملحوظة في معدلات تلوث الهواء ، ولعل أبرز ما يؤثر على جودة الهواء فيها هو الازدحام المروري الشديد ، فضلاً عن الزيادة السكانية وزيادة حركتها والأنشطة التي تؤثر سلباً على جودة الهواء

ولعل أبرز هذه الملوثات التي تؤثر على جودة الهواء PM2.5 هي جزيئات صغيرة جدًا تدور في الهواء ، يبلغ قطرها 2.5 ميكرومتر أو أقل ، ولها تأثيرات ملحوظة على صحة الإنسان ، حيث أنها تسبب أمراض القلب والجهاز التنفسي التي قد تؤدي إلى حتى الموت في بعض الحالات

يتضمن هذا المشروع الحديث عن جودة الهواء في مدن الضفة الغربية ، جنين ونابلس وطولكرم ، وكذلك إحدى قرى قرية عنزة.

في هذا المشروع ، وجدنا قيم PM2.5 على مدار 24 يومًا متواصلة ، باستخدام مستشعر AirU ، الذي يقيس كمية PM2.5 كل دقيقة وبدقة

بعد دراسة النتائج وتحليلها ، اتضح لنا أن منطقة جنين هي أكثر المناطق تلوثًا بجسيمات PM2.5 ، حيث كان متوسط ​​نسبة هذه الجزيئات خلال فترة الدراسة 40.05 ، تليها نابلس بمعدل 32.28 ، تليها طولكرم بمعدل 29.87 ، تليها قرية عنزة بمعدل 27.0

وبعد دراسة تأثير بعض العوامل الجوية والعوامل الطبيعية: سرعة الرياح وتأثير الرطوبة ودرجة الحرارة وارتفاع المكان والتساقط.

نحصل على النتائج التالية بخصوص سرعة الرياح ، فهناك علاقة مباشرة بينها وبين تركيز الملوث ، والعلاقة عكسية

أما الرطوبة النسبية فتوجد علاقة طردية بينها وبين تركيز الملوثات وعلاقة موجبة

أما ارتفاع المنطقة فلا علاقة بينها وبين تركيز الملوثات. بل إنه يؤثر على اختلاف درجات الحرارة ليلاً ونهاراً خلال الصيف والشتاء. نتيجة لذلك ، ليس لدرجات الحرارة علاقة واضحة بالتركيز العالي أو المنخفض للملوثات

أما تأثير المطر فكان واضحا حيث أن معدل التلوث في الأيام الممطرة كان أقل منه في الأيام غير الممطرة في المدن الثلاث.