An-Najah National University

Faculty of Engineering and Information Technology

Department of Energy and Environmental Engineering

Graduation Project 2:

“Determination of the optimum thickness of insulation for building walls in Nablus”

Prepared by:

Abeer Jayyousi

Supervisor:

Dr. Mohammed Alsayyed

A Graduation Project submitted to Department of Energy and Environmental Engineering as a fulfillment of the requirements for Bachelor degree in Energy and Environmental Engineering.

Dec,2019

Contents:

Table of figures:	II

Table of tables:	II

Dedication:	III

Acknowledgment:	IV

Abstract:	5

Introduction:	6

Literature Review:	7

Methodology:	8

Detailed calculations and data collection:	11

I.	Building Materials, Walls Structure, and Insulation Materials	11

II.	Cooling Load Estimation (CDDs method)	13

III.	Cooling Load Estimation (considering solar radiation calculations)	13

IV.	Optimal insulation thickness analysis Criterias:	15

LCC analysis calculations:	16

P1-P2 analysis calculation:	17

Results and discussion:	18

Conclusion and recommendations:	28

References	28

# Table of figures:

Figure 1: Methodology.	10

Figure 2: wall layers.	11

Figure 3: Insulation thickness effects on TC, RC, and IC.	18

Figure 4: sun's path.	19

# Table of tables:

Table 1: Average energy prices in Palestine.	7

Table 2: typical Palestinian walls construction and its thermal characteristics [6,12,13].	12

Table 3: Common insulation materials in Palestine.	13

Table 4: calculation parameters.	19

Table 5:Daily hourly solar radiation (n = 172-264).	20

Table 6: LCC results, Tb=18.	21

Table 7: P1-P2 results, Tb=18.	22

Table 8: LCC results, south.	23

Table 9: P1-P2 results, south.	24

Table 10: LCC results, north.	25

Table 11: P1-P2 results, north.	26

Table 12: LCC results, east.	27

Table 13: P1-P2 results, east.	28

Table 14: LCC results, west.	29

Table 15: P1-P2 results, west.	28

# Dedication:

To whom were always by my side… to every supporter among last four years and coming years… to my superheroes, Mom, Dad… to Mr. Walid Mizyed and the family of Reach Education Fund… to dreamers who couldn’t complete their high education due to life difficulties… to every teacher I met over my sixteen years of education… to every life-teacher… I dedicate this project.

# Acknowledgment:

To Allah, my first helper and supporter… To my family who struggled to let me reach this stage of life…to Reach Education Fund which was a main supporter… to my supervisor Dr. Mohammed Al-Sayyed and all of Energy and Environmental Engineering department, I give my deep hearted thanks!

# Abstract:

This research performs a comparison between two economical approaches in determining optimal insulation thickness for cooling season for buildings in Palestine, life cycle costing (LCC) and P1-P2. The analysis is done once without taking the heat gain from solar radiation incident into 4 oriented walls (S, N, E and W) and another time with considering it.

Buildings sector consumes 43% of total Palestinian energy. Although building insulation is a low investment with guaranteed beneﬁts alternative, it is still elective in Palestine. The city of Nablus was considered in the analysis. The analysis takes into consideration typical external walls construction, insulation types, weather data, electricity prices, cooling equipment conversion eﬃciencies, and market ﬁnancial parameters. Two types of insulation were studied which are polystyrene (denoted by I-1) and polyurethane (denoted by I-2). For energy type, electricity is assumed for summer cooling.

Optimal insulation thicknesses were calculated assuming 18 °C base temperatures Tb and Tsa as Tb when considering solar heat gain. Results show that insulation type have the highest signiﬁcant eﬀect on optimal insulation thickness. Polystyrene (I-1) and Polyurethane (I-2) investment costs are 83 and 171.43 $/m3, respectively. Despite that, same optimum ﬁnancial beneﬁts can be achieved using (I-2) at half thickness compared to (I-1). For all cases, optimal thickness varied between 2 and 7 cm which is still applicable.

The researcher believes that insulating buildings in Palestine will give a great push in energy conservation sector, and supports decision makers to develop a Palestinian building code that considers building insulation a mandatory item. Fortunately, results proved that it is ﬁnancially attractive.

# Introduction:

As fuel reserves is running out and so energy prices are getting higher all over the world, and due to climate changes caused basically by GHG emissions produced by burning fossil fuels, energy conservation became a trend for governments in building a strategic plan of energy management.

In the case of Palestine, where energy prices are one of the highest in the world, buildings consume 34,061.68 tera-joules per year which is around 43% of the total energy consumption [1]. But when considering electrical energy only, buildings consumed 3324 GWh in 2010, which forms 70% of the Palestinian national electrical consumption, this number is expected to double by 2020, and its proportion is expected to be the same [2].

Building sector is the fastest growing among other sectors and so is the most energy consuming.  in the last quarter of 2017, 2585 building licenses were issued, of course this number exclude the east side of Jerusalem which is under Israeli control, and the unregistered buildings inside Palestinian refugee camps [3]. As formal statistics from 2015, 82% of Palestinian families are consuming energy to heat their homes, where 39.4% of them depend on electricity, 25.4% depend on oil, 29.2% on wood, and 6% on other energy sources [4].

Palestinian authority, due to the fact that 95% of Palestine’s electricity is generated by Israeli generators and nearly similar percentage for other energy sources, aims to reduce energy consumption in order to, at least, fit the consumption with the available capacity. So, the Palestinian Energy Authority (PEA) developed a national energy efficiency action plan that aims to conserve 384 GWh between 2012 and 2020, 363 GWh (94.5% of total target) suggested to be in buildings [2].  Despite that, no serious action is done in the way of developing building codes to help reach those goals. For example, building insulation is still mandatory.

Building insulation helps in decreasing annual cooling and heating costs which leads to achieve governmental goals. Although it arises the initial cost of the building, but it is for sure economically feasible with nearly no maintenance costs and comparatively long life time especially with high costs of energy in Palestine where GDP per capita averaged 1379.31 USD from 1994 until 2016, which is about 16% of the worlds average [5].

Table 1 shows the average energy prices in Palestine.

Despite of the great savings that will occur if insulation is installed in Palestinian buildings, social awareness still needs to be improved. This could be difficult due to high initial cost, so the best decision would be building codes that penalizes defaulters.

Table 1: Average energy prices in Palestine.

| Energy type | Price |
| --- | --- |
| Electricity | 0.2 $/kWh |
| Diesel | 1.49 $/L |
| Gasoline 95 | 1.63 $/L |
| Coal | 1.81 $/kg |
| Kerosene | 1.43 $/L |
| LPG | 1.36 $/kg |

This research is considered to be an initiative step in designing building codes for Palestine. The case study of this research is the city of Nablus in cooling months (21 of June to 21 of September). The optimal thickness of insulation is calculated after taking the solar radiation incident on walls of the building, which is a main participant in the cooling load into consideration and calculating the solar air temperature to be the outside temperature. After that, the optimal thicknesses for southern, northern, western and eastern walls are calculated using two methods which are commonly used in determining the optimal thickness of insulation. The methods are Life Cycle Costing (LCC) and P1-P2. The results then are compared with those calculated without calculating the solar air temperature and just depending on ambient temperature and calculating CDDs.

# Literature Review:

Previous researches are rich of contributions related to buildings insulation optimization. Most of these contributions are based on the degree day method (DDM) such as [6], where the author developed a systematic procedure to define the optimal insulation thickness for heating purposes in Palestine using LCC approach. Similarly, [7] used the DDM to calculate the optimum insulation thickness for heating buildings in four climate zones in Turkey. Other researcher such as [8] studied the optimal insulation thickness using the DDM for summer cooling and winter heating applications in China, where P1-P2 economic model and LCC were applied. [9] determined the optimal insulation thickness for external walls and roof using DDM and applying a present worth analysis economic criterion, while [10] determined the optimal insulation thickness in different areas in Turkey using P1-P2 method. In all of the above-mentioned contributions, different insulation materials were analyzed and it was found that the optimal material type depends on cost, thermal conductivity, and weather conditions.

Another main contributor in this research is [11], the researcher estimated the solar air temperature and used it in estimating the cooling load and then calculating the optimal insulation thickness.

In all previous contributions, economic analysis was based on local market interest and inflation rates; there is a lack of researches that take escalation rate also into consideration when defining optimal insulation thickness. But, when talking about relatively long-life time projects, this interest rate becomes essential. In the same context, sensitivity analysis is very important in defining the optimal solution. This research presents a systematic approach for determining optimal insulation thickness for residential buildings in Palestine using both LCC and P1-P2 analysis, taking into consideration all affecting factor which are; wall construction, insulation type, weather data, energy prices, heating and cooling equipment conversion efficiency, and inflation.

The originality of this research lies behind the fact that it is a complementary work after Mohammed F. Alsayed and Rawan A. Tayeh from this university where they started with a comprehensive study on insulation optimization using LCC analysis that takes into consideration different Palestinian cities, in [12]. This research will also discuss results of P1-P2 analysis and compare with LCC’s and then take the solar radiation incident on building walls and compare all results. Moreover, since Palestinian economy is under international debts, and its governmental annual budget is based on international donations, efforts in reducing national energy consumption help in achieving Palestinians strategic independency objectives.

# Methodology:

To achieve the research objectives, the following methodology was adopted:

- Data was collected regarding walls structure, materials, thermal performance, local weather conditions, and market economic parameters. Also, market available insulations prices and thermal performance data was collected.

- CDDs at Tb=18 ֠C was calculated for Nablus using Equation 1.

- The direct solar radiation incident on different walls is calculated using Equation 3.

- Then Tsa is calculated using Equation 2 for South, North, East and West oriented walls and used as ambient temperature to calculate corrected CDDs.

- analysis economic parameters which are the PWF, combined interest rate P1 and P2 were calculated using Equation 12Equation 18 Equation 19, respectively.

- Installation and running costs are calculated using the first and second terms of  Equation 16Equation 20 for LCC method and also for P1-P2. Total costs are calculated using the same equations. Insulation thicknesses between 0.0 m to 0.3 were calculated, respectively.

- Optimal insulation thicknesses for all previous scenarios were calculated using Equation 17 for LCC method and Equation 21for P1-P2 method.

Figure 1: Methodology. is a diagram shows the detailed methodology of work.

Figure 1: Methodology.

# Detailed calculations and data collection:

## Building Materials, Walls Structure, and Insulation Materials

In general, buildings in Palestine are made of stones, concrete, bricks, and iron bars for reinforcement purposes. Despite the variety between regions, West Bank mountain regions buildings use stone and concrete walls while coastal regions and Jordan valley buildings use brick-based construction. Table 2 lists some typical wall structures for buildings in Palestine and their thermal resistance.

Table 3 shows the most common insulation materials used in Palestine and their thermal properties.

Figure 2: wall layers.

Table 2: typical Palestinian walls construction and its thermal characteristics [6,12,13].

| Wall type | Wall layers | Thickness (m) | Thermal conductivity (W/m.K) | Uov (W/m2.K) | Thermal resistance (m2. K/W) |
| --- | --- | --- | --- | --- | --- |
| External (I) | Stone Concrete Plaster | 0.07 0.20 0.03 | 1.70 1.75 1.20 | 2.77 | 0.361 |
| External (II) | External Plaster Concrete Internal Plaster | 0.02 0.20 0.03 | 1.20 1.70 1.20 | 2.98 | 0.336 |
| External (III) | External Plaster Hollow brick Internal Plaster | 0.02 0.20 0.03 | 1.20 0.90 1.20 | 2.25 | 0.444 |
| External (IV) | Stone Concrete Hollow brick Plaster | 0.07 0.20 0.07 0.03 | 1.70 1.75 0.90 1.20 | 2.28 | 0.438 |
| External (V) | Stone Concrete Air gap Brick Plaster | 0.07 0.20 0.05 0.07 0.03 | 1.70 1.75 0.28 0.90 1.20 | 1.62 | 0.617 |
| Roof | Asphalt mix Concrete Reinforced concrete Cement brick Plaster | 0.02 0.05 0.06 0.20 0.02 | 0.02 0.05 0.06 0.20 0.02 | 3.03 | 0.330 |

Table 3: Common insulation materials in Palestine.

| Type | Cost | Conductivity |
| --- | --- | --- |
| Polystyrene | 83 $ / m3 | 0.038 W / m K |
| Polyurethane rigid foam | 171.43 | 0.019 W / m K |
| Rock wool | 59.3 $ / m3 | 0.043 W / m K |

## Cooling Load Estimation (CDDs method)

This method is based on the principle of thermal equilibrium between building internal environment and its outside atmospheric temperature. The total number of annual cooling degree-days (CDDs) is calculated by Equation 1:

CDD=days(To-Tb)+

Equation 1

where Tb is the base temperature and To is the daily mean outdoor air temperature. The plus sign above the parentheses indicates that only positive values are to be counted. For Palestine weather conditions, Tb is taken to be 18.3 oC.

## Cooling Load Estimation (considering solar radiation calculations)

It should be noticed from Equation 1 that using CDDs method, solar radiation effects have not been considered. An alternative approach is to use the Tsa instead of the outside air temperature. It considers the incident solar radiation on a wall. So, the outside temperature should be higher by an amount equivalent to the effect of solar radiation. The solar air temperature is given by        Equation 2:

Tsa=To+∝sqsolasho-εσTo4-Tsky4ho

Equation 2

where To is the outside air temperature taken from local meteorological databases, αs the solar absorptivity of the surface, ho is the outer surface combined convection and radiation heat transfer coefficient, the summer design values of the ratio ∝s/ hofor light- and dark-colored surfaces are determined to be 0.026 and 0.052 m2 C/W, respectively, qs is the solar radiation incident on the surface, ε is the emissivity of the surface, σ is the Stefan-Boltzmann constant, and Tsky is sky temperature, ranging from a low of 230 K under a cold, clear sky to a high of approximately 285 K under warm, cloudy conditions [4]. For the sake of calculations, we will take Tsky as 275 K for hot, clear sky. After calculating the Tsa, CDD is calculated using        Equation 2 by assuming To equal to Tsa. [11,13]

The incident solar radiation qs, is calculated for a 90° tilted surface since we are calculating for building walls, just direct radiation is considered since the diffused radiation is neglected for clear summer sky. It is calculated four times four each wall type, for S, W, E, and N walls which means different azimuth angles. To calculate the solar radiation incident on a tilted surface, the following equation is used:

Id=I sin θsin α

Equation 3

sinθ=sinαcosβ+cosαcos(γs -γ)sinβ

Equation 4

Where Id is the direct solar radiation on a tilted surface, I is the direct part of a solar irradiance and taken to be 1000 W/m2as an average value, α is the elevation angle of the sun calculated using Equation 5, β is the tilt angle of the surface, γs is the sun’s azimuth angle calculated using Equation 6 and γ is the azimuth angle of the surface.[14]

α=90°-δ

Equation 5

sinγs =-sinωcosδ sinz

Equation 6

where δ is the declination angle, Equation 8, ω is the hour angle, Equation 9 and z is the zenith angle Equation 7. [11]

cosz=sinλsinδ+cosλcosδcosω

Equation 7

Where λ is the latitude of the location of interest,  δ is the declination angle, the angle between a plane perpendicular to a line between the earth and the sun and the earth's axis and determined from :

d = 23.45p / 180 * sin(2p * (284 + n) / 365)

Equation 8

Here, n is the day of the year.

ω=π(12-tsolar)12

Equation 9

Values are in radians and the solar time tsolar is found from:

tsolar= tlocal + Eqt60+(longlocal-longsm)15

Equation 10

Values are in hours, longlocal is the local longtude and longsm is the standard mean longitude.

There is an approximate formula for equation of time Eqt:

Eqt = -14.2 sin(π (n + 7) / 111)
for year day n between 1 and 106

Eqt = 4.0 sin(π (n - 106) / 59)
for year day n between 107 and 166

Eqt = -6.5 sin(π (n - 166) / 80)
for year day n between 167 and 246

Eqt = 16.4 sin(π (n - 247) / 113)
for year day n between 247 and 365

Equation 11

## Optimal insulation thickness analysis Criterias:

In this research, Life Cycle Costing (LCC) and P1-P2 method were used.

### LCC analysis calculations:

LCC present worth (PW) and adjusted interest rate (i*) are calculated using Equation 12 and Equation 13, respectively:

PW=IC+AS1+i*n-1i*1+i*n

Equation 12

i*=i+f+if

Equation 13

After calculating the PW, total cooling cost (CC) for insulated walls based on the number of cooling degree-days can be expressed using Equation 14:

CC=86400 CDD Ce PW Rt,w+xk COP+ Cix

Equation 14

Where Ce is the cost of electricity ($/m2), Rt,w is the total thermal resistance of the wall (m2 K / W) found from Equation 15, x is the insulation thickness (m)  and k is the thermal conductivity of the insulation (W/m2 K):

Rt,w=Ri+Rw+R0

Equation 15

Ri is the resistance of the inside air film (m2 K / W), Ro is the outside air film resistance (m2 K / W) and Rw is the total resistance of wall layers materials expect the insulation (m2 K / W).

Total cost is calculated using Equation 16  as follows:

TC=CC-Cix

Equation 16

where Ci is the initial cost of insulation material per unit volume including its labor cost ($/m2). Then, the optimal insulation thickness can be obtained by minimizing Equation 14 by taking the derivative of the total cost equation with respect to insulation thickness and set it equal to zero. As a result, optimal insulation thickness (xopt) for cooling in m is calculated using Equation 17.

xopt-cooling=86400 PW Ce CDDCOPkCi1/2-Rt,w k

Equation 17

### P1-P2 analysis calculation:

The idea of the method is that the life-cycle cost of a purchase option or alternative is calculated based on two economic indicators. The ﬁrst (P1) is the ratio of the life-cycle fuel cost to the ﬁrst year fuel cost. A low value of P1 indicates that immediate fuel costs are high and that consequently, potential immediate fuel savings are important. The second indicator (P2) is the ratio of life-cycle expenditures incurred as a result of the investment to the investment amount. A high value of P2 indicates that the investment has a low ﬁrst cost but higher costs over the life of the equipment

Equation 13Equation 14 are repeated to calculate adjusted interest rate and annual cooling load but here PW is replaced with P1, then P1 and P2 are calculated using Equation 18Equation 19:

P1N,i,f=j=1N1+ij-11+f0j = 1f-1 1-1+i1+fN       if i≠fN1+i                               if   i=d

Equation 18

P2=1+P1×Ms-Rv1+df-N

Equation 19

where Ms is the ratio of the annual maintenance and operation cost to the original ﬁrst cost, Rv is the ratio of the resale value to the ﬁrst cost. P2 can be taken as 1 because the maintenance and operation cost is zero.

Total cooling costs are calculated using Equation 20:

TC= CC×P1+P2×Cix

Equation 20

optimal insulation thickness is calculated with the same idea for LCC method so Equation 21 is used to calculate optimal insulation thickness (xopt) for cooling analysis in (m):

xopt-cooling=86400 P1 Ce CDDCOPkCi1/2-Rt,w k

Equation 21

Calculated optimal thickness is based on minimizing total cost (TC) which equals to the summation of initial cost (IC) and running costs (RC). However, these costs are functions of insulation thickness.         Figure 3 shows the variability of TC, RC, and IC with insulation thickness. It is obvious from         Figure 3 that the optimal insulation thickness can be defined as the minimum TC.

Figure 3: Insulation thickness effects on TC, RC, and IC.

# Results and discussion:

Following the research methodology, many parameters were defined based on local market conditions Error! Reference source not found. illustrates some of these parameters. Remaining parameters were presented in

Table 1

Table 3. The research takes the city of Nablus, which is located on the West Bank of Palestine as a case study. It studies the difference between results of optimal insulation thickness with taking the heat gain of wall coming from incident solar radiation on walls during the day to calculate

the solar air temperature, and without and just taking a base temperature equals 18 °C. The analysis is applied on summer days  (21/06 to 21/09) only, since the solar radiation is a heating parameter and so not an additional load in heating season.

The daily solar radiation incident on S, N, E and W oriented walls for the analysis period is calculated and results are plotted and tabulated in Error! Reference source not found..

Then total costs, annual savings and optimal thicknesses for both cases are calculated using the both preferred economical approaches, LCC and P1-P2 assuming two different common types of insulation in Palestine local market, Polystyrene and Polyurethane. Results are plotted and tabulated in Table 6 to Table 15.

When talking about the result of both economic approaches in general, results shows different and nearly 1 cm higher values of optimal thicknesses for different wall types using P1-P2 for the five main cases. Also, the comparison between insulation types, shows that using polyurethane as an insulation material requires nearly the half thickness of insulation rather than using polystyrene. This is because the thermal conductivity of polyurethane is nearly the half of that for polystyrene as shown in Table 3.

After taking the solar heat gain into consideration, results optimal insulation thickness, taking wall type no. 6 as a sample and using the economical approach of LCC, were different for each oriented wall. The highest thickness was for eastern wall with 4.75 cm optimal polystyrene thickness, then comes the western wall with 4.63 cm optimal polystyrene thickness which is the same as results for usual CDD’s method, both can be rounded to 5 cm, after that comes the southern wall with 3.73 cm optimal polystyrene thickness and finally comes the northern wall with 2.41 cm optimal polystyrene thickness. As mentioned before, P1-P2 results are a pit more and polyurethane thickness is halved.

This difference between thicknesses for different oriented walls is due to the sun path during summer as shown in Figure 4 and so different amount of heat gain.

Table 4: calculation parameters.

| Ri | 0.06 (m2. oC / W) |
| --- | --- |
| Ro | 0.12 (m2. oC / W) |
| i | 10% |
| f | 3% |
| e | 3% |
| N | 20 years |
| COP | 3 |
| latitude | 32.22 ֠ |
| longitude | 35.12 ֠ |

Figure 4: sun's path.

Table 5:Daily hourly solar radiation (n = 172-264).

Daily hourly solar radiation (n = 172-264)

Table 6: LCC results, Tb=18.

|  | LCC |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

Table 7: P1-P2 results, Tb=18.

|  | P1-P2 |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

Table 8: LCC results, south.

|  | LCC |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

Table 9: P1-P2 results, south.

|  | P1-P2 |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

Table 10: LCC results, north.

|  | LCC |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

Table 11: P1-P2 results, north.

|  | P1-P2 |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

Table 12: LCC results, east.

|  | LCC |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

Table 13: P1-P2 results, east.

|  | P1-P2 |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

Table 14: LCC results, west.

|  | LCC |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

Table 15: P1-P2 results, west.

|  | P1-P2 |  |
| --- | --- | --- |
|  | I-1 (polystyrene) | I-2 (polyurethane) |
| Total cost ($/m2) |  |  |
| Annual savings ($/year) |  |  |
| Optimal thickness (cm) |  |  |

# Conclusion and recommendations:

Optimal insulation thickness was calculated for different insulation types, oriented walls and wall types for buildings in Palestine, Nablus. Results show that insulation type has a significant effect on the optimal thickness, thus, polyurethane insulation is recommended because it reduces the required thickness to almost half. Varying the azimuth angle of the wall also affected the required thickness slightly; however, it is not practical to choose the required thickness in advance during the building construction phase based on future occupants expected behaviour. Thus, it is suggested that PEA should initiate awareness raising campaign, to convince buildings occupants to consider a reasonable heating and cooling equipment usage behaviour.

Achieved results should not in any case assumed accurate as 100%, since weather conditions vary annually, assumed COP vary with equipment load, and the most importantly that almost all previous contributors ignored, that DD’s method assumes that the building will be occupied 24 hours per day, 365 days per year, obviously, this is not the case for almost all buildings. Despite that, achieved results give a very good idea how many cm’s the optimal thickness should be. Luckily, total cost will not vary a lot when moving slightly around the optimal value. To spread these results and make it practical for the local market engineers, it is suggested to round results to the closest upper 0.5 cm, this is realistic and applicable for LCC and P1-P2 methods. The results will not be that different from using CDD or corrected CDD if we rounded.

# References

[1] PCBS, Palestinian Central Bureau of Statistics. Palestinian Energy Balance in Physical Units, 2016. Available at: http://www.pcbs.gov.ps/Portals/_Rainbow/Documents/Energy-11a-2016.html, access data: 31/05/2018.

[2] PEA. The Palestinian National Energy Efficiency Action Plan (NEEAP). 2010.

[3] PCBS, Palestinian Central Bureau of Statistics. Main Indicators for Building Sector Licenses, fourth quarter, 2017. Available at: http://www.pcbs.gov.ps/Portals/_Rainbow/Documents/BuildingLic/a_main_BLic_2017q4.htm. access data: 31/05/2018.

[4] PCBS, Palestinian Central Bureau of Statistics. Annual Statistics. Available at: http://www.pcbs.gov.ps/site/lang__ar/706/Default.aspx, access data: 31/05/2018.

[5] Palestine Monetary Authority PMA. Annual Report 2016. available at:

http://www.pma.ps/Portals/1/Users/002/02/2/Publications/English/Annual%20R

eports/PMA%20Annual%20Reports/Annual_Report_2016.pdf. access date: 01/06/2018.

[6] Hasan A. Optimizing insulation thickness for buildings using life cycle cost. Appl Energy 1999;63:115–24.

[7] Bolatturk A. Determination of optimum insulation thickness for building walls with respect to various fuels and climate zones in Turkey. Appl Therm Eng 2006;26:1301–9.

[8] Yu J, Yang C, Tian L, Liao D. A study on optimum insulation thicknesses of external walls in hot summer and cold winter zone of China. Appl Energy 2009;86:2520–9.

[9] Sisman N, Kahya E, Aras N, Aras H. Determination of optimum insulation thicknesses of the external walls and roof (ceiling) for Turkey’s different degree-day regions. Energy Policy 2007;35:5151–5.

[10] Ucar A, Balo F. Determination of the energy savings and the optimum insulation thickness in the four different insulated exterior walls. Renew Energy 2010;35(1):88–94

[11] Mohammed J. Al-Khawaja. Determination and selecting the optimum thickness of insulation for buildings in hot countries by accounting for solar radiation. Applied Thermal Engineering 24 (2004) 2601–2610

[12] Mohammed F. Alsayed, Rawan A. Tayeh. Life cycle cost analysis for determining optimal insulation thickness in Palestinian buildings. Journal of Building Engineering 22 (2019) 101-112.

[13] M. Alsaad, M. Hammad. Heating and Air Conditioning for Residential buildings. 5th edition, 2013. University of Jordan, Amman.

[14] F. Jaugscha, M.-O. Löwner. Estimation of solar energy on vertical 3D building walls on city quarter scale. The International Archives of the Photogrammetry, Remote Sensing and Spatial Information Sciences, Volume XLII-2/W2, 2016 11th 3D Geoinfo Conference, 20–21 October 2016, Athens, Greece.