Abstract

Sheep MilkPro is a design and implementation of a feeding machine for sheep and cows. It consists of four units: a Heating Unit, an Input/Output Unit, making milk, and a Control Unit. To keep the water boiling, the heating appliance uses a heating element. Users of the input/output unit can prepare milk using an LCD and a keypad. Using an Arduino Mega and an ESP8266, the Control Unit establishes communication between the components and links the machine to the mobile application. It’s mainly for water and milk stocks. The water is heated to 40 degrees to protect these animals from health problems.

Milk production occurs through a mixing process between them, after which special valves are opened so that animals obtain milk through suction.

The milk is passed through flow sensors so that the owner of the barn can obtain accurate information about the quantities that are consumed in each feeding, which is what matters to him from an economic point of view. The machine also has a cleaning feature, which is a process of pumping water without milk to remove residues and plankton from food.

It also contains additional features such as Ultrasonic sensors that are used to get an alert of low water and milk supply, RFID for each sheep to check the main information about it, and the RTC for the scheduling process, to carry out the feeding operation or cleaning operation at specific times.

Sheep MilkPro

هو تصميم وتنفيذ آلة تغذية للأغنام والأبقار. تتكون من أربع وحدات: وحدة التسخين، وحدة

الإدخال/الإخراج، صنع الحليب، ووحدة التحكم. للحفاظ على غليان الماء، يستخدم الجهاز عنصر تسخين. يمكن لمستخدمي وحدة الإدخال/الإخراج إعداد الحليب باستخدام شاشة LCD ولوحة مفاتيح. باستخدام Arduino Mega و ESP8266، تنشئ وحدة التحكم الاتصال بين المكونات وتربط الآلة بالتطبيق المحمول. إنها مخصصة بشكل أساسي لمخزون الماء والحليب. يتم تسخين الماء إلى 40 درجة لحماية هذه الحيوانات من المشاكل الصحية.

يتم إنتاج الحليب من خلال عملية خلط بينهما، بعد ذلك تُفتح الصمامات الخاصة بحيث تحصل الحيوانات على الحليب من خلال الشفط.

يمر الحليب عبر مستشعرات التدفق بحيث يمكن لصاحب الحظيرة الحصول على معلومات دقيقة حول الكميات التي يتم استهلاكها في كل عملية تغذية، وهو ما يهمه من الناحية الاقتصادية. تحتوي الآلة أيضًا على ميزة التنظيف، وهي عملية ضخ الماء بدون حليب لإزالة البقايا والبلانكتون من الطعام.

كما تحتوي على ميزات إضافية مثل مستشعرات الموجات فوق الصوتية التي تُستخدم للحصول على تنبيه عند انخفاض مخزون الماء والحليب، RFID لكل غنم للتحقق من المعلومات الرئيسية عنها، و RTC لعملية الجدولة، لتنفيذ عملية التغذية أو التنظيف في أوقات محددة.