BIKE CAMPUS HUB ABSTRACT

Abstract BikeCampusHub is a smart bike rental station system designed for

students at university campuses. With the long distance between campuses, this

project aims to provide an efficient and convenient mode of transportation for

students, which was inspired by European bike rental systems. The system contains

a smart bike, a connected vest wirelessly, and rental stations that communicate

continuously. Students have two ways to access bikes either by using their student

ID or by receiving a verification code on their mobile phones. The system tracks

essential student information, including phone numbers, student IDs, names, and

rental status, ensuring that a student cannot rent another bike until the previous one

is returned. Additionally, students can view all stations on their phones to check

their availability, whether they have bikes, and whether they are active or

deactivated by the admin. The system also provides real-time updates on the

remaining time to return a rented bike. The admin has full control and visibility

over the system through a dedicated dashboard and website, allowing them to

monitor all student activities, manage bike rentals, track the location of each

student via GPS on a real-time map, control rental timing for stations, and

deactivate or activate the stations. The Firebase database serves as the centralised

system for storing and managing all data. The smart bike is equipped with an RFID

system and communicates with the connected vest using ESP modules configured

as an access point and client. The system also includes directional detection to

activate LED lights, enhancing safety by indicating when the bike stops or changes

direction. Additionally, a locking mechanism provides security by locking the bike

at the station when the user doesn’t return it on time or when the user goes outside

of the college boundaries. BikeCampusHub offers a practical solution for student

transportation within the university, combining convenience, security, and smart

technology to improve campus mobility.