Abstract

Sheep MilkPro is a design and implementation of a feeding machine for sheep and cows. It consists of four units: a Heating Unit, an Input/Output Unit, making milk and a Control Unit. To keep the water boiling, the heating appliance uses a heating element. Users of the input/ output unit can prepare milk using an LCD and a keypad. Using an Arduino Mega and an ESP8266, the Control Unit establishes communication between the components and links the machine to the mobile application.

It’s mainly for water and milk stocks. The water is heated to 40 degrees to protect these animals from any health problems. Milk production takes place through a mixing process between them, after which special valves are opened so that animals obtain milk through the process of suction.

The milk is passed through flow sensors so that the owner of the barn can obtain accurate information about the quantities that are consumed in each feeding, which is what matters to him from an economic point of view. The machine also has a cleaning feature, which is a process of pumping water without milk to remove residues and plankton from food. It also contains additional features such as Ultrasonic sensors that are used to get an alert of low water and milk supply, RFID for each sheep to check main information about it, and the RTC for the scheduling process, to carry out the feeding operation or cleaning operation at specific times.

Sheep MilkPro

هو تصميم وتنفيذ آلة تغذية للأغنام والأبقار. تتكون الآلة من أربع وحدات: وحدة تسخين، وحدة إدخال/إخراج، وحدة إنتاج الحليب، ووحدة تحكم. للحفاظ على الماء في حالة الغليان، تستخدم 
وحدة التسخين عنصر تسخين.

ولوحة تحكم.   (LCD) يمكن لمستخدمي وحدة الإدخال/الإخراج تحضير الحليب باستخدام شاشة 
ربط    تقوم وحدة التحكم بتأسيس التصال بين المكونات و(ESP8266) أردوينو ميجا و باستخدام

الالة بالتطبيق المحمول.

يتم تسخين الماء إلى 40 درجة لحماية هذه الحيوانات من أي مشاكل صحية. يتم إنتاج الحليب من خلال عملية خلط بين الماء والحليب، ثم يتم فتح صمامات خاصة لكي تحصل الحيوانات على الحليب من خلال عملية الشفط. يتم تمرير الحليب عبر مستشعرات التدفق لكي يتمكن صاحب الحظيرة من الحصول على معلومات دقيقة حول الكميات التي يتم استهلاكها في كل عملية تغذية، وهو ما يهمه من الناحية الاقتصادية. كما تحتوي الآلة على ميزة التنظيف، وهي عملية ضخ الماء بدون حليب لإزالة البقايا والعوالق من الطعام

كما تحتوي على ميزات إضافية مثل مستشعرات الموجات فوق الصوتية التي تُستخدم للحصول على تنبيه   عند انخفاض كمية الماء والحليب، 
 لكل شاة للتحقق من المعلومات الرئيسية عنها(RFID)

لعملية الجدولة، لتنفيذ عملية التغذية أو عملية التنظيف في أوقات محددة(RTC(