# Chapter Two

# Preliminary Design

# CH.2 Slab & Beam Design:

Slab system and dimensions:

The deflection is the most important factor that controls the slab thickness. According to ACI 318 – 02 code

Table (2.1): Minimum thickness of one way ribbed slab & beam.

| Minimum Thickness (h) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Member | One end continuous | Two end continuous | simply supported | Cantilever |
| Beam or ribbed one way slab | Ln/18.5 | Ln/21 | Ln/16 | Ln/8 |
| Solid one way slab | Ln/24 | Ln/28 | Ln/20 | Ln/10 |

# 2.1 Block A:

Design of slab:

Fig (2-1): Slab for banqueting hall

Thickness of slab:

t=L18.5  One end continuous

=5.5*100/18.5

=30 cm.

# Design one way Ribbed slab:

Fig (2-2): Rib slab for banqueting hall

Width of rib=55cm

Load=DL+LL

Dead load/rib =slab weight+ block weight+ own weight+ super imposed

DL= (0.06*2.4*0.55) + (0.4*24*1.4) + (0.24*0.15*2.4) + .55 *0.324                =0.478ton/m

LL=0.55*0.4=0.22 ton/m

𝔀u=1.2DL+1.6LL

𝔀u=1.2*0.487+1.6*0.22=0.926ton/m

bE=40+15=55cm center to center

bE=l/4=6104/4=151cm

bE=16t+bw=16*30+15=495cm

take minimum bE

bE=55cm

𝓜from SAP:

Fig ( 2-3):𝓜 for ribbed slab

𝓜-ve=3.1ton.m

𝓜+ve=1.87ton.m

Assume d=26cm

# Design for M-ve

𝓜df=0.9* bE*a*0.85* 𝒇c΄(d-a/2)

𝓜df=0.9*.55*.06*.85*240*(.26-(.06/2))/103

𝓜df=.9*55*6*.85*240*(26-(6/2))/103 =13.94ton.m

𝓜df>𝓜u =3.1 ton

a>ℎf

b=55	   ⟨rectangular section⟩

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌 = .85*240/42000(1-(1-((2.35*3.1*105)/(.9*55*262*240)).5))

𝜌 =0.0023

As = 𝜌*b*d= .oo23*55*26=2.431cm2

*𝜌 min=14.06/ℱy=.0033

*𝜌min=0.8 𝒇c΄/ ℱy

*𝜌 min=.8*(240.5)/42000=.003

As-ve = ρ* bw* d=.003*15*26=1.17cm2 <   As ok

cb=6120d/6120+ℱy=6120*26/6120+4200=15.42cm

ab=.85Cb=.85*15.42=13.1cm

Tb=Cb

.85*240*55*13.1=As b*4200

As b=35cm2

As b>As>As min

35>2.431>1.17

ф12

As=3.29

3ф12

Checks:

1) bmin :

bmin =2*c + 2*ds + n*db + (n-1)*S +1.

de=ℎ-ys.

ys =c+ds+0.5*db.   For one layer

bmin = 2*3+2+2*1.4+1*3+1=14.8<15cm

d=ℎ-ys

ys=3+1+.5*1.4=4.7cm

de =25.3cm

2) DR& ф

ℰt = (d-c/c)*.003

ℰt= (25.3-1.35/1.35)*.003

.05>.004 no Double Rein & ф=0.9 ok

C=a/.85=1.15/.85=1.35

ℎf>a=4200*2*1.54/.85*240*55=1.15cm

3) 𝓜d =0.9*2*1.54*4200*(25.3-(1.15/2))/105

𝓜d =2.88ton.m>2.39ton.m    ok

Check shear for rib

𝓥u=1.15*wu*ln/2 = 1.15*0.712*(6.04-0.6)/2=2.23ton

Ø𝓥c=1.1*.075*.53*(2400.5) *15*25.3/103=2.57ton

𝓥u<Ø𝓥c   no shear reinforcement

# Design for M+ve

𝓜df=13.94ton>1.87

𝓜df>𝓜u

a<ℎf

𝜌 = .85*240/42000(1-(1-((2.35*1.87*105)/(0.9*55*262*240)).5))

𝜌 =0.0013

As = 𝜌*b*d= 0.0013*55*26=1.93cm2

𝜌min=14.06/ℱy=.0033

𝜌min =.8*(240.5)/42000=.003

As min= 𝜌min * bw*d=.003*15*26=1.17cm2< As ok

Cb = 15.472cm

Asb = 35cm2 > as    steel yield

2ф12 one layer

# Checks:

- bmin = 2*3+2+2*1.2+1*3+1=14.4<15cm

Ys=3+1+.5*1.2=4.8cm

de =30-4.6=25.4cm

2) ℰt = (d-c/c)*.003

ℰt = (25.4-1/1)*.003

.073>.004 no d.r & Ø=0.9 ok

C=a/0.85=0.85/0.85=1.35

ℎf>a=4200*2*1.13/.85*240*55=0.85cm

3) 𝓜d =0.9*2*1.13*4200*(25.3-(0.85/2))/105

𝓜d =2.13 ton.m>1.87 ton.m    ok

# Design for main girder

Fig (2-4) Dimension Of main beam

wu=1.2DL+1.6LL

DL = slap own weight + weight of the block +imposed load +self weight of rib

=0.06*1*2400+2*0.4*0.24*1.4+0.324+0.2*0.24*2.4=0.852 ton/m2

Imposed load=0.4ton/m2

wu=1.2*0.852+1.6*0.4=1.66 ton/m

wu  On girder =Wu (2.55+3.02) +1.2own weight.

Own weight=2.95 ton/m.

wU=1.66(5.57) +1.2*2. 59=12.35 ton.

ℎ=𝓛/18.5=23.6/18.5=127.6cm     ℎ=130cm.

d=120cm

bw =70cm

bE =Ln/4=2360/4=590cm

=16t+bw =16*30+70=550cm

=center to center= (2.55+3.05) =557cm

Take min bE=550cm

Own weight= (0.7*0.9+0.3*1.5)*2.4=2.59

From SAP:

Fig (2-5):𝓜 for gairder

𝓜+ve=wu*Ln212

=12.35*23.62/12=573.2ton.m

𝓜-ve=wu*Ln224

=12.35*23.62/24=286.6ton.m

# Design for M+ve=573.2ton.m

Assume d=120

bw=70cm

bE=150cm

𝓜df=0.9*150*30*0.85*240(120-30/2)/105

=867.51ton.m

𝓜df>𝓜ua<ℎf

𝜌 =0.85*240/4200(1-√1-(2.35*573.2*105/0.9*150*1202*240)

𝜌 =0.0076

𝜌min=14.06/ℱy=0.003

=0.8√𝒇c΄/ ℱy=0.003

=4/3 𝜌calculated=0.01 𝜌min=0.003

As min= 𝜌min*bw *d=0.003*70*120=25.2cm2

As= 𝜌* bE*d=0.0075*150*120=136.8 cm2

As> As minok

Ф28    As= (23 ф28)

Cb=6120*120/ (6120+4200) =71.16

ab=0.85 Cb=60.5cm

Tb=Cb As b=0.85*240*150*60.5/4200=440.8 cm2

As b> As  steel yield

Check:-

1)  3 layer (8+8+7)   ф stirrup=12mm

bmin =2*3+2*1.2+8*2.8+7*3+1=52.8cm<70cmok

Ys (one layer)=6+1.2+0.5*2.8=8.6cm

Ys (3  layer)= Ys (one layer)+(n2(2.5+db)+n3(2*2.5+1.5 db)+n1(0))/#  of bars

=8.6+ (8(2.5+2.8) +7(5+1.5*2.8))/23=13.24

d=h- Ys =130-15.24=116.76cm<120cm ……ok

2) DR& ф

ℰt= ((d-c)/c)*0.003

a=4200*23*6.16/0.85*240*150=19.45cm<30cm ….. ok

c=a/0.85=22.88cm

ℰt=((116.76-22.88)/22.88)*0.003=0.012

ℰt>0.004

No D.R & Ø=0.9

3) 𝓜d=0.9*4200*23*6.16*(116.76-19.45/2)/105

=573.23ton.m   ….ok

# Design for -veM=286.6 ton.m

d=120cm

bw=70cm

bE=150cm

𝓜df=867.51ton.m

a<ℎf

𝜌 =0.85*240/4200(1-√1-(2.35*286.6*105/0.9*150*1202*240)

𝜌=0.0036

As= 𝜌* bE*d=0.0036*150*120=64.8cm2

𝜌min=0.003

As min= 𝜌min*bw*d=0.003*70*120=25.2 cm2

As> As minok

Ф11 As= (23 ф28)

Check:-

1)  2 layer (6+5)   ф stirrup=12mm

bmin=2*3+2*1.2+6*2.8+5*3+1=41.2cm   <70cm    ok

Ys (one layer) =6+1.2+0.5*2.8=8.6cm

Ys (2 layer) =8.6+ (5(2.5+2.8) +2.8)/11

d=h- Ys=130-11=119cm d=119cm

2) DR& ф

ℰt= ((d-c)/c)*0.003

a=4200*11*6.16/0.85*240*150=9.3cm

c=a/0.85=9.3/0.85=10.94cm

ℰt= ((119-10.94)/10.94)*0.003=0.03 >0.004

No D.R & ф=0.9

𝓜d =0.9*4200*11*6.16*(119-9.3/2)/105=292.89ton.m

Shear for Girder:

Fig (2-6): Shear for Girder

𝓥u/ ф≤ Vc+2.1√ 𝒇c΄*bw*d

𝓥c=0.53√ 𝒇c΄* bw*d

𝓥u=wu2 = 12.35*23.6/2=145.73ton

145.73/0.75≤2.63√240*70*120/103

194.3≤342.25   ok for Dimension

ф Vc=0.75*0.53√240*70*120/103

=51.73ton

𝓥u> ф 𝓥c   need shear reinforcement

𝓥u/ ф=145.73/0.75=194.3=Vd

𝓥u(at d=120)  194.3/1180=Vn/(1180-120)  𝓥u=174.54

𝓥c=69 ton          𝓥c/2=34.5 ton

𝔁=34.5*1180/194.3=209.5cm

𝓥s=174.54-69=105.54

фstirrup=10mm               Avleg=0.79cm2

S=Av*ℱy*d/Vs

S=2*0.79*4200*120/105.54*103=7.6cm

Smin=8cm S=8cm

For SmaxS≤ (Av*ℱy/3.5bw)=2*0.79*4200/3.5*70*103=27cm

𝓥s≤√ 𝒇c΄* bw*d

105.54≤√240*70*120*10-3

105.54≤130.13 ok

Smax≤60cm

≤d/2    120/2 =60cm

≤27.1cm

Take     S=25cm

Find the distance

𝓥=𝓥c+Av*Fy*dS

=69+2*0.78*120*4200/25*103

=100.45 ton

𝔁=100.45*1180/194.3=610cm

610-209.5=400.5 cm

(5)(15@8=120cm)(430cm)(17@25=425cm)(8@25=200cm)

Fig (2-7 ): Shear for Gairder

Slab thickness for waffle slab

Two way waffle slab:

Fig (2-8 ): Two way waffle slab

Assume α>2

H=Ln(0.8+0.2 ℱy/2800)/(36+9β)

=8.75(0.8+0.2*4200/2800)/(36+9*(8.75/6.04)

=19 cm   ≊ 20 cm

Check if α > 2:

α=IbIs

Edge beam L-section

Fig ( 2-9): Edge beam L-section

Assume

bw=60 cm

h=70 cm

hf=20 cm

For L- section

beff= min of { bw+4 hf} or { bw+(h- hf)}  beff=110 cm

Moment of inertia for L-section

I=0.0225 m4

Interior beam T-section:

Fig ( 2-10): Interior beam T-section

Assume

bw=60 cm

h=70 cm

hf=20 cm

For T- section

beff= min of { bw+8 hf} or { bw+2(h- hf)}  beff=160 cm

Moment of inertia for T-section

I=0.0263 m4

Values of α:

Frame A:

B1-B2

Islab= (1/12) (5.57) (0.2)3=3.713*10-3 m4

α= 7.08

Frame C:

B3-B4

Islab= (1/12) (4.09) (0.2)3=2.727*10-3 m4

α= 8.25

Frame B:

B5-B6

Islab= (1/12) (6.775) (0.2)3=4.517*10-3 m4

α= 5.8

Frame D:

B7-B8

Islab= (1/12) (4.015) (0.2)3=2.677*10-3 m4

α= 8.4

α avg=(8.4+5.8+8.25+7.08)

=7.38 >2   ok

Dimension of waffle and γequ and equivalent thickness in sap:

Waffle dimension (60*60*25)

I solid=I waffle

(1/12)(0.2)3(0.75) = (k/12) (0.15) (h)3

5*10-4 m4 ≤ I waffle

K=1+(0.2* beff / bw)

K=1+(0.2*0.75/0.15)

=2

If we take h=33 cm  I waffle=8.56*10-4  m4 > 5*10-4   m4  ….. Ok

Banqueting hall

Fig ( 2-11):Waffle slap

I solid=I waffle

(1/12)(h)3(0.75)= 8.56*10-4

h=24 cm

To find γequ:

Weight waffle/ m2

= ((0.75*0.75*0.08+0.75*0.15*0.25+0.15*0.25*0.6)*2.4)/0.752

=0.408t/m2

Weight solid/m2=weight waffle/ m2

γequ*0.24=0.408

γequ=1.7 ton/m3

# 2.2 Block B:

Beams & Slabs thickness:

Beam & slab @ elevation 4m:

One way

Simply supported L/16 =1167.63 =73 cm.

One end continuous L/18.5 =1206.4/18.5 =65.2 cm.

Both ends continuous L/21 =1154/21 =55 cm.

Take h=75 cm. d=70cm   & b=d/1.75 b=40 cm.

Two ways:

One end continuous L/18.5 = 868.13/18.5=47 cm.

Both ends continuous L/21 = 807/16=33.8

Take h=60cm. d=55 cm     & b=d/1.75 b=35 cm.

One way slabs:

h both end continuous=L/21 =493/21=23.5 cm

h one end continuous =L/18.5 =22.5 cm.

Two way slabs:

h=Ln(0.8+0.2 ℱy/2800)/(36+9β)

=8.75(0.8+0.2*4200/2800)/(36+9*1)

=20 cm.

Fig (2-12) Two way ribbed slab

Fig (2-13) Two way ribbed slab

Check if α > 2:

α=Ib/Is

Ib=(1/12)(0.3)(0.6)3

=6.3*10-3

Is

Values of α :

B1-B2

Islab=(1/12)(6.13)(0.2)3=4.09*10-3 m4

α1= 7.08

B3-B4

Islab=(1/12)(3.63)(0.2)3=2.42*10-3 m4

α2= 2.6

B5-B6

Islab=(1/12)(5.13)(0.2)3=3.42*10-3 m4

α4= 5.16

B7-B8

Islab=(1/12)(2.15)(0.2)3=1.43*10-3 m4

α3= 4.14

B9-B10

Islab=(1/12)(3)(0.2)3=2*10-3 m4

α5= 3.15

α avg=( α1+ α2+ α4+ α5)/4

=3.11 >2   ok

Calculation for hequ & γequ:

Take h=25 cm.

One way:

Fig (2-14) One way ribbed slab

Isolid=Irib

(1/12)(0.65)(h)3=3.371*10-4

hequ=0.195 m  hequ=20 cm.

Weight/m2=0.4*0.17*1*0.55+0.15*0.17*1*2.4

+0.55*0.08*1*2.4

=0.2047 ton/m2

γequ* hequ =γ*h

γequ= (0.2042/0.55)/0.2 =1.9 ton/m3

Two ways:

Weight/m2=0.4*0.17*0.4*1*0.55+0.08*0.552*2.4+0.15*0.17 *0.55*2.4+0.15*0.17*0.4*2.4=0.1312 ton/m2

γequ= (0.1312/0.55)/0.2 =2.17 ton/m3

Beam & slab @ elevation 8m:

For beam:

h=60cm.

d=55 cm

b=35 cm.

For slab:

hequ=20cm.

γequ=2.17 ton/m3

Beam & slab @ elevation 8.5 m:

For beam:

h=80cm.

d=75 cm

b=45 cm.

For slab:

hequ=20cm.

γequ=1.9 ton/m3

Beam & slab @ elevation 9 m:

For beam:

h=60cm.

d=55 cm

b=35 cm.

For slab:

hequ=20cm.

γequ=2.17 ton/m3

Beam & slab @ elevation (12,13,16)m:

For beam:

h=60cm.

d=55 cm

b=35 cm.

For slab:

hequ=20cm.

γequ=2.17 ton/m3

# 2.3 Block C:

Thickness of slab:

A      t=L28        two end cont

=2.18/28= 7. 8cm

t=L24  one end cont

=2.18/24=9.1cm

B       t=L20  simply supported beam

=1.7/20=8.5 cm

C         t=L28   simply supported beam

=2.9/20=14.5cm

D        t=L28   two end cont

=5.1/28=18cm

t=L24  one end cont

=6.96/24=29cm

E       t=L28  two end cont

=6.1/28=28cm

t=L28   one end cont

=3.6/24=15cm

F       t=L28     two end cont

=3.8/28=14cm

t=L28   one end cont

=4.4/24=19cm

Take ℎ slab=30 cm

Wu=1.2D+1.6L

DL=Super imposed load +Own weight

DL⇒SIL=1*1*.003*2.6+0.02*2.3+0.1*2=0.324 ton/m2

OW=0.3*2.4*1*1=0.72 ton/m2

LL=0.5 ton/m2

Wu=1.2*(0.324+0.72)+1.6*0.5=2.053 ton/m

# Design of slab:

Fig(2-15):Slabs of Block C

Section A:

Fig(2-16):𝓜 for slab A

𝓜+ve=0.36 ton.m

𝓜-ve=0.89 ton.m

Design for 𝓜+ve=0.29 ton.m

ℎ=30 cm   , c=ℎ-d-0.5*db, d=ℎ-1.8-0.5*db

c=4cm      ⇒d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*0.36*105/.9*262*100*240)

𝜌=0.0001

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

Use 𝜌=𝜌min=0.0021

As=𝜌*b*d=0.0021*100*26=5.46cm2

Ф12       As=   (5ф12)   S=20 cm

Checks:

- c=ℎ-d-0.5*db

d=ℎ-1.8-0.5*db

d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 3ℎ)

S=20cm  …ok

Shrinkage & Temperatures:

𝜌sh    ⇒ ℱy (2800-3500)    ⇒𝜌sh=0.002

⇒ ℱy (4200)              ⇒𝜌sh=0.0018

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12), S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 5ℎ)

S=20< (45 or 5*30) cm  …….  OK.

Design for 𝓜-ve=0.89 ton.m

ℎ=30 cm   , c=ℎ-d-0.5*db, d=ℎ-1.8-0.5*db

c=4cm      ⇒d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*0.89*105/.9*262*100*240)

𝜌< 𝜌min

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

Use 𝜌=𝜌min=0.0021

As=𝜌*b*d=0.0021*100*26=5.46cm2

Ф12       As=   (5ф12)   S=20 cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 3*30cm)

S=20cm        OK

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12) ,  S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30)cm .......ok

Section B

Fig (2-17 ):𝓜 for slab B

𝓜+ve=0.25 ton.m

𝓜-ve=0.494 ton.m

Design for 𝓜+ve=0.25 ton.m

ℎ=30 cm   , c=4cm      ⇒d=30-4=26cm

𝜌=.85𝒇c/ℱy*(1-√ (1-2.35𝓜u/фbd2 𝒇c))

𝜌=.85*240/4200(1-√ (1-2.35*0.25*105/.9*262*100*240)

𝜌<𝜌min

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

Use 𝜌=𝜌min=0.0021

As=𝜌*b*d=0.0021*100*26=5.46cm2

Ф12       As=   (5ф12)   S=20 cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size  & ≤ (45cm or 3ℎ)

S=20cm  …..ok

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12), S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30) cm ……….ok

Design for 𝓜-ve=0.494 ton.m

ℎ=30 cm   , c=4cm      ⇒d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*0.494*105/.9*262*100*240)

𝜌< 𝜌min

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

Use 𝜌=𝜌min=0.0021

As=𝜌*b*d=0.0021*100*26=5.46cm2

Ф12       As=   (5ф12)   S=20 cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 3*30cm)

S=20cm  ….ok

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12), S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30) cm    OK.

Section C:

Fig (2-18 ):𝓜 for slab C

𝓜+ve=0.744 ton.m

𝓜-ve=1.49 ton.m

Design for 𝓜+ve=0.744 ton.m

ℎ=30 cm   , c=4cm      ⇒d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*0.744*105/.9*262*100*240)

𝜌<𝜌min

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

Use 𝜌=𝜌min=0.0021

As=𝜌*b*d=0.0021*100*26=5.46cm2

Ф12       As=   (5ф12)   S=20 cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 3ℎ)

S=20cm  ……ok

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12), S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30) cm    ……ok

Design for 𝓜-ve=1.49 ton.m

ℎ=30 cm   , c=4cm      ⇒d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*1.49*105/.9*262*100*240)

𝜌< 𝜌min

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

Use 𝜌=𝜌min=0.0021

As=𝜌*b*d=0.0021*100*26=5.46cm2

Ф12       As=   (5ф12)   S=20 cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 3*30cm)

S=20cm   ……ok

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12) ,  S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30) cm  ……ok

Section D:

Fig (2-19 ):𝓜 for slab D

𝓜+ve=8.412 ton.m

𝓜-ve=8.808 ton.m

Design for 𝓜+ve=8.412 ton.m

ℎ=30 cm   ,c=4cm      ⇒d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*8.412*105/.9*262*100*240)

𝜌=.0034

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

𝜌> 𝜌min ……ok

As=𝜌*b*d=0.0034*100*26=8.86 cm2

Ф12         As   (9ф12), S=11cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 3*30cm)

S=11cm ……ok

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12), S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30) cm ……ok

Design for 𝓜-ve=8.81 ton.m

ℎ=30 cm   , c=4cm      ⇒d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*8.81*105/.9*262*100*240)

𝜌=.0036

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

𝜌> 𝜌min  ……ok

As=𝜌*b*d=0.0036*100*26=9.36 cm2

Ф12         As   (9ф12), S=11cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 3*30cm)

S=11cm  ……ok

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12), S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30) cm ……ok

Section E:

Fig (2-20 ):𝓜 for slab D

𝓜+ve=3.88 ton.m

𝓜-ve=5.64 ton.m

Design for 𝓜+ve=3.88 ton.m

ℎ=30 cm   , c=4cm      ⇒d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*3.88*105/.9*262*100*240)

𝜌<𝜌min

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

Use 𝜌=𝜌min=0.0021

As=𝜌*b*d=0.0021*100*26=5.46cm2

Ф12       As=   (5ф12)   S=20 cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 3ℎ)

S=20cm ……ok

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12), S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30) cm    OK.

Design for 𝓜-ve=5.64 ton.m

ℎ=30 cm   , c=4cm      d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*5.64*105/.9*262*100*240)

𝜌=.0022

𝜌min=0.0018*h/d=0.0018*30/26=0.0021

𝜌> 𝜌min ……ok

As=𝜌*b*d=0.0022*100*26=5.72 cm2

Ф12         As   (6ф12), S=16.7cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

&≤ (45cm or 3*30cm)

S=11cm  ……ok

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*ℎ =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12),  S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30) cm ……ok

# Section F:

Fig (2-21 ):𝓜 for slab F

𝓜+ve=1.72 ton.m

𝓜-ve=3.5 ton.m

Design for 𝓜+ve=1.72 ton.m

ℎ=30 cm   , c=4cm      ,d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*1.72*105/.9*262*100*240)

𝜌<𝜌min

𝜌min=0.0018* ℎ /d=0.0018*30/26=0.0021

Use 𝜌=𝜌min=0.0021

As=𝜌*b*d=0.0021*100*26=5.46cm2

Ф12       As=   (5ф12)   S=20 cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

Smax   ≥4/3 Max aggregate size

&≤ (45cm or 3 ℎ)

S=20cm ….ok

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*h =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12), S=20 cm

Design for 𝓜-ve=3.5 ton.m

ℎ=30 cm   , c=4cm   ,d=30-4=26cm

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*3.5*105/.9*262*100*240)

𝜌< 𝜌min

𝜌min=0.0018*ℎ/d=0.0018*30/26=0.0021

Use 𝜌=𝜌min=0.0021

As=𝜌*b*d=0.0021*100*26=5.46cm2

Ф12       As=   (5ф12)   S=20 cm

Checks:

- d=30-1.8-0.5*1.2=27.6 cm

c=30-27.6 -0.5*1.2=1.8 cm

- Spacing between bars

Smax   ≥4/3 Max aggregate size

&≤ (45cm or 3*30cm)

S=20cm        OK

Shrinkage & Temperatures:

AS/Sh= 𝜌sh*b*d =0.0018*100*30

AS/Sh=5.4 cm2

AS/Sh       (5ф12), S=20 cm

Spacing between bars

S  ⇒   ≥4/3 Max aggregate size

S=20< (45 or 5*30) cm    OK.

Shear Reinforcement for slab:

𝓥u<ф𝓥c

No need for Sh.R

# Design of Beams

Fig ( 2-22) Beams n Block C

# Design beam A

𝔀u=2.053*1.09*2

=4.48 ton/m.

Hand Calculation:

𝓜+ve=𝔀u*𝓛2/24=4.48*6.32/24=7.41 ton.m

𝓜-ve=𝔀u*𝓛2/12=4.48*6.32/12=14.82 ton.m

From SAP:

Fig ( 2-23):𝓜 for Beam A

𝓜+ve=7.28 ton.m

(7.41-7.28)/7.41*100%=1.8%<10% OK

𝓜-ve=14.82 ton.m

Design 𝓜-ve=14.82 ton.m

Hidden beam h=30 cm, assume d=26 cm, ρmin=.0033

𝝆max=.0184, 𝝆 =.0103

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝝆=.85*240/4200*(1-√ (1-2.35*𝓜u/0.9*b*262*240))=.0103

b=𝓜u/0.236   ,b=65 cm.

𝝆=.85*240/4200(1-√ (1-2.35*14.82*105/.9*262*65*240)

𝝆=0.01           𝝆max>𝝆new>𝝆min     ok.

As=𝝆*b*d = 0.01*65*26 = 16.9cm2.

Ф16        As=     (9ф16).

𝝆n=As/ (bd) = 9*2.01/ (65*26) = 0.0107.

Checks:

1) Bmin=2*c + 2*ds + n*db + (n-1)*S +1.

de=ℎ-ys.

ys =c+ds+0.5*db. For one layer

Where:    c: cover

ds: Diameter of stirrup. =8mm

n: Number of Bars.

db: Diameter of Bars. =16mm

S: spacing between two bars. =30mm

de: Effictive depth

ys: Depth to the center of As.

bmin =2*3+2*.8+9*1.6+8*3+ 1 =47cm<65cm   ⇒OK

ys =3+1+0.5*1.6=4.8 cm

de =30-4.8=25.2cm   <26cm ok

2)    Double reinforcement & ф

ℰt = (de-c)/c*0.003≥0.005.

C=a/0.85      a=Fy*As*(β*𝒇c*b)

a=4200*9*2.01/ (0.85*240*65) =5.73cm

C=5.73/0.85=6.75cm.

ℰt = (25.2-6.75)/6.75*0.003=8.2*10-3 >0.004

ф=0.9 & No D.RE

3)   𝓜d = ф*As*ℱy*(de-a/2)

= 0.9*9*2.01*4200*(25.2-5.73/2)*10-5

𝓜d = 15.27ton.m>14.82ton.m    ….OK.

Design for 𝓜+ve=7.41ton.m

𝝆=0.85*240/4200*(1-√ (1-2.35*7.41*105/.9*240*65*262)).

𝝆 =0.0047         𝝆 max> 𝝆 new> 𝝆 min     ok.

As= 𝝆 *b*d = 0.0047*65*26 = 7.943cm2.

Ф16        As=     (4ф16).

𝝆 n=As/ (bd) = 4*2.01/ (65*26) = 0.00476 OK.

Checks:

1)    Bmin=2*c + 2*ds + n*db + (n-1)*S +1.

de=ℎ-ys.

ys =c+ds+0.5*db.   For one layer

bmin =2*3+2*.8+4*1.6+3*3+ 1 =24cm<65cm   ⇒OK

ys =3+1+0.5*1.6=4.8 cm

de =30-4.8=25.2cm   <26cm ok

2)    Double reinforcement & ф

ℰt = (de-c)/c*0.003≥0.005.

c=a/0.85,    a=Fy*As*(β*𝒇c*b)

a=4200*4*2.01/ (0.85*240*65) =2.55 cm

c =2.55/0.85=3 cm.

ℰt = (25.2-3)/3*0.003=22*10-3 >0.004

ф=0.9 & No D.RE

3)   𝓜d = ф*As*ℱy*(de-a/2)

= 0.9*4*2.01*4200*(25.2-2.55/2)*10-5

𝓜d = 7.27 ton.m

𝜟= (7.41-7.27)/7.41*100%=1.89 %< 3% OK within the range.

# Shear Reinforcement for Beam A:

Ф8 Ds & S min=8 cm

Hand Calculation:

𝓥u=𝔀u*𝓛/2

= 4.48*6.3/2=14.11 ton.

From SAP:

Fig (2-24): Shear diagram for Beam A

𝓥u=14.11 ton.

de=25.2cm,   b=65 cm.

𝓥d=𝓥u/ф=14.11/0.75=18.81 ton

𝓥d/315=𝓥n/ (315-de)

18.81/315=𝓥n/ (315-25.2)

𝓥n=17.31 ton

𝓥c=0.53*√𝒇c*bw*de

=0.53*√240*65*25.2*10-3=13.45 ton

𝓥u>ф𝓥c    need shear reinforcement.

𝓥c/2=13.45/2=6.73 ton

𝔁2/6.73=315/18.81 , 𝔁2=112.7 cm =113 cm

For de=25.2 cm   (18.81-17.31) ton

𝓥s=17.31-13.45=3.86 ton

S=Av*ℱy*de/𝓥s=1*4200*25.2*10-3/3.86=27.42 cm

𝓥s≤√𝒇c*bw*de

3.86 ton ≤√240*65*25.2*10-3 = 25.4 ton        OK

S=min of ≤60 cm

≤d/2   =25.2/2=12.6 cm

≤27.4

Take S=12 cm

For S max   𝓥s min

S≤Av*Fy/ (3.5*bw) =1*4200/ (3.5*65) =18.4 cm

Take S max=18 cm.

The point at which we will stop extending S max=18 cm

𝓥s min=𝓥c + Av*ℱy*de/S

=13.45+1*4200*25.2*10-3/18=19.33 ton

Use ф8@12 cm.

Fig ( 2-25): Shear diagram for Beam A

# Check section capacity:

𝓥u/ф≤𝓥c+2.1√𝒇c*bw*de

17.31/0.75≤13.45+2.1√240*65*25.2*10-3

18.81 ton ≤ 66.74 ton   Section is ……ok

# Design of beam F:

From SAP & Hand Calculation

Fig ( 2-26): 𝓜 for Beam F

𝓜-ve = 22.21 ton.m

𝓜 +ve = 11.26 ton.m

Hidden beam h=30 cm, assume d=26 cm, 𝝆min=.0033

𝝆max=.0184,    take  =.0103

Design For 𝓜-ve = 22.21 ton.m

𝝆=.85𝒇c /Fy*(1-√ (1-2.35𝓜u/фbd2 𝒇c))

𝝆=.85*240/4200*(1-√(1-2.35*𝓜u/0.9*b*262*240))=.0103

b=𝓜u/0.236   ,b=94.11 cm.

b=100 cm.

𝝆 =.85*240/4200(1-√ (1-2.35*22.21*105/.9*262*100*240)

𝝆 =.0096           𝝆 max> 𝝆 new> 𝝆 min     ok.

As= 𝝆 *b*d = 0.0096*100*26 = 24.96 cm2.

Ф22        As=     (7ф22).

𝝆 n=As/ (bd) = 7*3.8/ (100*26) = 0.0102

Checks:

1) bmin =2*c + 2*ds + n*db + (n-1)*S +1.

de=ℎ-ys.

ys =c+ds+0.5*db. For one layer

bmin =2*3+2*.8+7*2.2+6*3+ 1 =42 cm<100cm   ⇒…..ok

ys =3+1+0.5*2.2=5.1 cm

de =30-5.1=24.9 cm   <26cm …..ok

2)    Double reinforcement & ф

ℰt =(de-c)/c*0.003≥0.005.

c=a/0.85      a=Fy*As*(β*𝒇c*b)

a=4200*7*3.8/(0.85*240*100)=5.48cm

c =5.48/0.85=6.44 cm.

ℰt = (24.9-6.44)/6.44*0.003=8.6*10-3 >0.004

⇒ф=0.9 & No D.RE

3)   𝓜d = ф*As*ℱy*(de-a/2)

= 0.9*7*3.8*4200*(24.9-5.48/2)*10-5

𝓜d = 22.28 ton.m > 22.21 ton.m    …..ok

Design for  +ve = 11.26 ton.m

𝝆=0.85*240/4200*(1-√ (1-2.35*11.26*105/.9*240*100*262)).

𝝆 =0.0046         𝝆 max> 𝝆 new> 𝝆 min     ok.

As= 𝝆 *b*d = 0.0046*100*26 = 11.96 cm2.

Ф16        As=     (6ф16).

𝝆 n=As/ (bd) = 6*2.01/ (100*26) = 0.0046 …..ok

Checks:

1)    bmin =2*c + 2*ds + n*db + (n-1)*S +1.

de=ℎ-ys.

ys =c+ds+0.5*db. For one layer

bmin =2*3+2*.8+6*1.6+5*3+1 =33.4 cm<100cm  …ok

ys =3+1+0.5*1.6=4.8 cm

de =30-4.8=25.2cm   <26cm ….ok

2)    Double reinforcement & ф

ℰt = (de-c)/c*0.003≥0.005.

c=a/0.85,    a=Fy*As*(β*𝒇c*b)

a=4200*6*2.01/ (0.85*240*100) =2.48 cm

c =2.48/0.85=2.92 cm.

ℰt = (25.2-2.92)/2.92*0.003=23*10-3 >0.004

ф=0.9 & No D.RE

3)   𝓜d = ф*As*ℱy*(de-a/2)

= 0.9*6*2.01*4200*(25.2-2.48/2)*10-5

𝓜d = 10.92 ton.m

𝜟= (11.26-10.92)/10.92*100%=3 %< 3% …..ok within the range.

# Shear Reinforcement for Beam F:

Ф8 Ds & S min=8cm

Hand Calculation:

𝓥u=20.26 ton

From SAP

Fig ( 2-27): Shear diagram for Beam F

𝓥u=20.26 ton.

de =24.9cm ,   b=100 cm.

𝓥d = 𝓥u/ф=20.26/0.75=27 ton.

𝓥d/331=𝓥n/ (331-de)

27/331=𝓥n/ (331-24.9)

𝓥n=25 ton

𝓥c=0.53*√𝒇c*bw*de

=0.53*√240*100*24.9*10-3=20.45 ton

𝓥u>ф𝓥c    need shear reinforcement.

# Check section capacity:

𝓥u/ф≤𝓥c+2.1√𝒇c*bw*de

20.26/0.75≤20.45+2.1√240*100*24.9*10-3

27 ton ≤ 101.45 ton   Section is OK.

𝓥c/2=20.45/2=6.73 ton

𝔁2/10.2=331/27 ,𝔁2=125 cm

For de=24.9 cm       𝓥 between (27-25) ton

𝓥s=25-20.45=4.6 ton

S = Av*ℱy*de/𝓥s=1*4200*24.9*10-3/4.6=22.7 cm.

𝓥s≤√𝓥c*bw*de

4.6 ton ≤√240*100*24.9*10-3 = 25.4 ton   …..ok

S=min of   ≤60 cm

≤d/2   =24.9/2=12.5 cm

≤22.7

Take S=12 cm

For S max   𝓥s min

S≤Av*ℱy/ (3.5*bw) =1*4200/ (3.5*100) =12 cm

Take S max=12 cm.

Use ф8@12 cm.

Fig (2-28): Shear diagram for Beam F

# Design of beam E:

Fig (2-29): Dimension of Beam E

# T-Section

𝔀u beam=𝔀u slap +own weight

ℎ=𝓛/16   simply supported

ℎ=10.45/16=65.3cm

ℎ = 𝓛 /18.5=10.7/18.5=58 cm .  One way end cont

Take h=65cm

Assume bw= 35cm

be= 𝓛 n/4=10.45/4=261cm

= bw +16t=35+16*30= 515 cm

Center to center = 3/2+6. 76/2=4.88m

Assume

be =60cm

𝔀u beam=2.053*(6.76/2+3/2) +1.2*0.35*0.35*2.4

=10.37 ton/m

From SAP

Fig (2-30 ): M for Beam E

Hand Calculation:

𝓜+ve=𝔀u*𝓛n2/12 =10.37*10.452/12=94.37 ton.m

𝓜-ve= 𝔀u*ln2/24= 47.21 ton.m

Design for 𝓜+ve=94.37 ton.m

𝝆min = .003

𝝆max=.0183

Assume d=58cm, bw =35 cm, be=60 cm

𝓜 d. flange=ф*be*t*β* 𝒇c΄*(d-t/2)

𝓜 df=0.9*60*30*0.85*240*(58-30/2)*10-5

=142.1 ton.m    ,a<hf

𝓜 u< 𝓜 df

𝝆=0.85*240/4200*(1-√ (1-2.35*94.37*105/0.9*240*60*582)).

𝝆 =0.0145         𝝆 max> 𝝆 new> 𝝆 min     ok.

As= 𝝆 *be*d = 0.0145*60*58 = 50.46 cm2.

Ф26        As=     (10ф26).  Two layer (5+5)

As min= 𝝆 min*bw*d=0.003*35*58=6.09 cm2

As>As min    …..ok

cb =6120*d*(6120+ ℱ y)

cb  =6120*58/ (6120+4200) =34.4 cm

ab =cb*0.85=34.4*0.85=29.24 cm

Tb=Cb    --- Asb=0.85*𝒇c*be*ab/ℱy

Asb=0.85*240*60*29.24/4200=85.21 cm2

Asb>As

Checks:

1)    Bmin=2*c + 2*ds + n*db + (n-1)*S +1.

de=ℎ-ys.

ys =c+ds+0.5*db. For one layer

bmin =2*3+2*.8+5*2.6+4*3+ 1 =33.6cm<35cm   ⇒OK

ys =3+1 +0.5*2.6=5.3 cm  one layer

ys2=ys1+ (2.5+db)*n2/n   two layer

de =65-7.85=57.15cm   <58 cm ok

2)    Double reinforcement & ф

ℰt = (de-c)/c*0.003≥0.005.

c=a/0.85,    a= ℱ y*As*(β* 𝒇c΄*b)

a=4200*10*5.31/(0.85*240*60)=18.22 cm

c=18.22/0.85=21.44 cm

ℰt = (57.15-21.44)/21.44*0.003=5*10-3 >0.004

ф=0.9 & No D.RE

3)   𝓜d = ф*As*ℱy*(de-a/2)

= 0.9*10*5.31*4200*(57.15-18.22/2)*10-5

𝓜d = 96.42 ton.m   >𝓜u=94.37 ton.m

Design for 𝓜-ve=47.2 ton.m

Assume d=58cm, bW =35 cm, be=60 cm

𝓜d.flange=ф*be*t*β*𝒇c*(d-t/2)

𝓜df=0.9*60*30*0.85*240*(58-30/2)*10-5

=142.1 ton.m    , a<hf

𝓜u<𝓜df

𝝆=0.85*240/4200*(1-√ (1-2.35*47.2*105/0.9*240*60*582)).

𝝆 =0.0066         𝝆 max> 𝝆 new> 𝝆 min   …..ok

As= 𝝆 *be*d = 0.0066*60*58 = 23.08 cm2.

Ф26        As=     (5ф26).  One layer

As min= 𝝆 min*bw *d=0.003*35*58=6.09 cm2

As>As min    …..ok

cb =6120*d*(6120+ℱy)

=6120*58/ (6120+4200) =34.4 cm

ab =cb*0.85=34.4*0.85=29.24 cm

Tb=Cb ----Asb=0.85*𝒇c*be*ab/ℱy

Asb=0.85*240*60*29.24/4200=85.21 cm2

Asb>As

Checks:

1)    Bmin=2*c + 2*ds + n*db + (n-1)*S +1.

de=ℎ-ys.

ys =c+ds+0.5*db.  For one layer

bmin =2*3+2*.8+5*2.6+4*3+ 1 =33.6cm<35cm   …..ok

ys =3+1+ 0.5*2.6=5.3 cm  one layer

de =65-5.3=52.7 cm   < 58 cm ok

2)    Double reinforcement & ф

ℰt = (de-c)/c*0.003≥0.005.

c=a/0.85,    a=ℱy*As*(β*𝒇c*b)

a=4200*5*5.31/ (0.85*240*60) = 9.11 cm, a<hf …..ok

c=9.11/0.85=10.72 cm

ℰt = (52.7-10.72)/10.72*0.003=12*10-3 >0.004

ф=0.9 & No D.RE

3)   𝓜d = ф*As*ℱy*(de-a/2)

= 0.9*5*5.31*4200*(52.7-9.11/2)*10-5

𝓜d = 48.32 ton.m   >𝓜u=47.2 ton.m

# Shear Reinforcement for Beam E:

Fig (2-31 ): Shear diagram for Beam E

Ф8 DRs  & S min=8cm

Hand Calculation:

𝓥u=𝔀u*𝓛/2=10.37*10.45/2=54.18 ton

From SAP

𝓥u=54.18 ton

d=58cm,   be=60 cm, bw =35 cm

𝓥d=𝓥u/ф=54.18/0.75=72.24 ton

𝓥d/523=𝓥n/ (523-58)

72.24/523=𝓥n/ (523-58)

𝓥n=64.23 ton

𝓥c=0.53*√𝒇c*bw *de

=0.53*√240*35*58*10-3=16.67 ton

𝓥u>ф𝓥c    need shear reinforcement.

# Check section capacity:

𝓥u/ф≤𝓥c+2.1√𝓥c*bw *de

54.18/0.75≤16.67+2.1√240*35*58*10-3

72.24 ton ≤ 82.71 ton   Section is …..ok

𝓥c/2=16.67/2=8.33 ton

𝔁2/8.33=523/72.24 ,𝔁2=60.3 cm

For de=58 cm       𝓥 (64.23 - 16.67) ton

𝓥s=64.23-16.167=47.56 ton

S=Av*ℱy*de/𝓥s=1*4200*58*10-3/47.56=5.12 cm

Take S=8 cm.

𝓥s≥√𝒇c*bw *de

47.56 ton ≥√240*35*58*10-3 = 31.45 ton    …..ok

S=min of   ≤30 cm

≤d/4   =58/4=14.5 cm

≤8

Take S=8 cm

For S max   𝓥s min

S≤Av*ℱy/ (3.5*bw) =1*4200/ (3.5*35) =34.3 cm

𝓥s≥√𝓥c*bw* de

47.56 ton ≥√240*35*58*10-3 = 31.45 ton    …..ok

S=min of   ≤30 cm

≤d/4   =58/4=14.5 cm

≤34.3 cm

Take S max=14 cm.

The point at which we will stop extending S max=18 cm

𝓥s min=𝓥c + Av*𝓥y*de/S

=16.67+1*4200*58*10-3/14=34.07 ton

𝔁1/34.07=523/72.24     ,𝔁1=246.7 cm

(5cm)(12@8=96cm)(151cm)(14@14=196cm) (3@25=75cm)

Fig ( 2-32): Shear diagram for Beam E

# Design of beam D:

𝓥u=2.053*0.5*6.22

=7.144ton/m

From SAP:

Fig (2-33 ): 𝓜 for Beam D

𝓜+ve=17.29 ton.m

𝓜 -ve =8.55 ton.m

From hand calculation:

𝓜 +ve=𝔀𝓛2/8-(17.27+15.52)/2

= 8.69 ton.m

𝓜 -ve=17.27 ton.m

Design for 𝓜 -ve=17.27 ton.m

Hidden beam=30cm

d=26cm

𝝆min=0.0033

𝝆max=0.75ρb=0.0184

Assume 𝝆=0.18 𝒇c΄/ℱy

=0.0103

𝝆 max> 𝝆 > 𝝆 min    …..ok

𝝆 < 𝝆 b steel yield

1/b= ((1-(ρℱy/0.85 ф 𝒇c΄) 2) (фd2 𝒇c΄/2.35𝓜u)

b=𝓜u/0.236=17.27/0.236=73.2 cm

b=75 cm

𝝆 new=0.85*240/4200(1-√1-(2.35*105*17.27/0.9*75*262*240))

𝝆 new =0.1

As= 𝝆 *b*d

=0.01*75*26=19.5 cm2

Ф16   As= (10 ф16)

𝝆 =10*2.01/75*26

=0.0103

Checks:

1) For bmin

bmin =2*c + 2*ds + n*db + (n-1)*S +1.

de=ℎ-ys.

ys =c+ds+0.5*db. For one layer

bmin =2*3+2*0.8+10*1.6+9*3+1=51.6cm<75cm  …..ok

ys =3+1+0.5*1.6=4.8cm

de =30-4.8=25.2cm <26cm   …..ok

2) Double reinforcement & ф

ℰt = (de-c)/c*0.003≥0.005.

c=a/0.85      a=ℱy*As*(β*𝒇c*b)

a =10*2.01*4200/0.85*240*75=5.52cm

c=a/0.85=6.5cm

ℰt = ((25.2-6.5)/6.5)*0.003

=8.63*10-3

ℰt >0.005  ф=0.9

ℰt >0.004No D.R

3)   𝓜d = ф*As*ℱy*(de-a/2)

𝓜d= (0.9*10*2.01*4200/105)(25.2-5.52/2)

=17.05 ton.m

((17.27-17.05)/17.27)*100%=1.27 %< 3%   ok

Design for 𝓜+ve=8.69ton.m

Hidden beam=30 cm

d=26cm

b=75cm

𝝆min=0.0033

𝝆max=0.75ρb=0.0184

𝝆 =0.85*240/4200(1-√1-(2.35*105*8.69/0.9*75*262*240))

=0.0048

𝝆max>𝝆> 𝝆min    …..ok

As=𝝆*b*d

As=0.0048*75*26

=9.36cm2

Ф16       As= (5 ф16)

𝝆 new=5*2.01/75*26=0.0052

Checks:

1)    bmin =2*c + 2*ds + n*db + (n-1)*S +1.

de= ℎ -ys.

ys =c+ds+0.5*db. For one layer

bmin =2*3+2*0.8+5*1.6+4*3+1

=28.6cm <75 cm ok

ys =4.8

de =25.4cm<26cm   ok

2)    Double reinforcement & ф

ℰt = (de-c)/c*0.003≥0.005.

c=a/0.85,    a=ℱy*As*(β* 𝒇c΄*b)

c= (4200*5*2.01/0.852*240*75)

=3.25 cm

a=2.76 cm

ℰt = ((25.2-3.25)/3.25)*0.003

=0.021

ℰt >0.005  ф=0.9

ℰt >0.004No D.R

3)   𝓜d = ф*As*ℱy*(de-a/2)

𝓜d= (0.9*5*2.01*4200/105) (25.2-2.76/2)

=9.08 ton.m > 8.69ton.m   …..ok

# Shear Reinforcement for Beam D:

Ф8 Ds & S min=8cm

From SAP:

Fig ( 2-34): Shear diagram for Beam D

𝓥u=19.26 ton

𝓥n=𝓥u/0.75

𝓥n=19.26/0.75=25.68 ton

𝓥c=0.53√ 𝒇c΄* bw*d

𝓥c=0.53*√240*75*26/103

𝓥c=16.01 ton    ф 𝓥c=0.75*16.01=12.01 ton

ф 𝓥c<𝓥u  need shear reinforcement

𝓥n at d=25.4 cm

25.68/270=𝓥n/ (270-25.4)

𝓥n=23.26

𝓥c=/2=16.01/2=8.015

25.68/270=8.015/𝔁1

𝔁1=84.22 cm

𝔁 2=270*16.01/25.68

=168.33

Use ф 8mm for stirrup

Av=2*Afor one leg

=1 cm2

𝓥s=𝓥n-𝓥c

=23.2616.01=7.25ton

S= (Av*ℱy*d)/ 𝓥s

=1*4200*25.4/7.25*102=14.7 cm>8cm …..ok

√ 𝒇c΄ *b*d=√240*75*25.4*10-3=29.51 ton

𝓥s≤√ 𝒇c΄*b*d

S≤d/2 =25.4/2=12.7cm

S≤60cm

S=14.7cm

Take the smallest    S=12 cm

𝓥s min= (left 𝓥c/2)

𝓥s min=3.5bwd

S ≤ Av*ℱy/3.5bw=1*4200/3.5*75=16 mc

S ≤ 60cm

S ≤ d/2=12.7 mc

Take S max=12 cm.

Find the point at which we will stop extending S=12 cm

𝓥s=Av*ℱy*d/S

=1*4200*25.4/12*103=8.89 ton

𝓥n=𝓥s+𝓥c

=8.89+16.01=24.9 ton

25.68/270=24.9/𝔁3 𝔁 3=262 cm

(5cm)(16@12 =197cm)(@25 =68cm)

Fig ( 2-35): Shear diagram for Beam D