AutoXchange (Exchanger Machine)
Short Summary

Based on the graduation project report (June 2025)

AutoXchange is an automated currency exchanger machine that performs real-time conversion between Jordanian Dinar (JOD) and Israeli Shekel (ILS). The system combines hardware, an AI banknote-recognition model, and a secure web application to deliver fast and safer self-service exchange transactions.

Key Features

Real-time conversion (JOD ↔ ILS) using live exchange-rate data.

User access via QR code (opens the web app) plus mandatory RFID ID-card verification before any transaction.

AI-based banknote verification using a camera and a custom-trained CNN under controlled lighting (>95% accuracy in testing).

Cash handling with rollers and a conveyor belt that routes accepted notes to the correct storage bin; early-stage cancel returns notes (before first storage).

Dispensing via a vacuum pickup head on a Y-Z gantry for banknotes, plus a servo-driven 1 ILS coin dispenser to cover fractional payouts (1-19 ILS).

Web app (Next.js + Supabase) with authentication, current rate display, transaction execution, digital receipts, transaction history, and a digital wallet for fractional remainders.

Chatbot assistant (Chatbase) to guide users and reduce mistakes.

Testing Highlights and Limitations

Testing reported smooth end-to-end operation (from QR scan to dispensing) with a typical transaction time around 4 minutes, AI classification accuracy above 95% in the enclosed camera box, and overall conversion accuracy above 80%. Main constraints were budget and time: only one coin module (1 ILS) was used instead of multiple denominations, and advanced counterfeit detection (e.g., UV/magnetic sensors) was not implemented, relying on camera-based AI.

Recommended Future Improvements

Add more coin dispensers (e.g., 5 ILS, 10 ILS) to speed payouts and reduce mechanical wear.

Improve the vacuum pickup system (stronger motor, replaceable seals) to reduce occasional suction loss.

Add low-cost physical counterfeit sensors (UV or magnetic-ink) to strengthen fraud detection.

Cache exchange rates locally for short intervals to handle API/network outages.

Extend toward ATM-like functionality (deposit/withdraw) and consider coin input support.