[image: image20.bmp]
An-najah  National  University
Faculty of Engineering

Computer Engineering Department

Project Report
Home Security System

Prepered BY : Helwa Dabak.

Advance to: DR.Raed Alqadi .
                    DR.Luai  Malhis .
                    EN.Haya Samaneh .
***Aim of the Project:***
         Main aim is a Security for more than one building ,it detect intruder if the window open or an cutting to the  infrared signal in the door ,each flat has its own detection sensor if such as intruder detection happened it will automatically  set up call to specific number and then when answer the call play recorded sound to inform the place of  the intruder, also In the project interface in the PC indicate to the place of the action and save the action time ,date and place in the log file in the PC where user can retrieve these info. 
***Main Component in the  Project:***

1~ Two Pic 16F877.

2~ LCD .
3~ Real Time Clock  DS1302 .
4~ Infrared Circuit .

5~ sound modem .

***Project in Steps:***
(1).Build basic circuit for  the two building.
     I build the basic circuit to both pic ,the following is the Schematic for basic circuit .
[image: image1.png]


[image: image2.emf]Title


Size


Document Number


Rev


Date:


Sheet


of


<Doc>


<RevCode>


<Title>


E


1


1


Saturday, April 05, 2008


RE0/RD/AN5


8


RE1/WR/AN6


9


RE2/CS/AN7


10


GND


12


OSC2/CLKOUT


14


RC0/T1OSO/T1CKI


15


RC1/T1OSI/CCP2


16


RC2/CCP1


17


RC3/SCK/SCL


18


RD0/PSP0


19


RD1/PSP1


20


RD2/PSP2


21


RD3/PSP3


22


RC4/SDI/SDA


23


RC5/SDO


24


RC6/TX/CK


25


RC7/RX/DT


26


RD4/PSP4


27


RD5/PSP5


28


RD6/PSP6


29


RD7/PSP7


30


GND


31


MCLR/VPP


1


OSC1/CLKIN


13


RA0/AN0


2


RA1/AN1


3


RA2/AN2


4


RA3/AN3/VREF


5


RA4/TOCKI


6


RA5/AN4/SS


7


RBO/INT


33


RB1


34


RB2


35


RB3


36


RB4


37


RB5


38


RB6


39


RB7


40


VDD


11


VDD


32


U3


PIC16f877


SW1


SERIAL SOCKET


C4


0.1u


1


2


J1


Input Voltage


S8


Program


X1


4MHz


D1


1N4007


C8


12p


C9


12p


R1


33K


C12


6.8u


S9


RESET


C11


0.1u


C10


0.1u


D2


LED


C3


1u


C5


1u


C6


1u


VIN


1


GND


2


VOUT


3


U1


Regulator


C7


1u


5V


C1


10u


C2


0.1u


R1OUT


12


T1OUT


14


T2OUT


7


T2IN


10


T1IN


11


GND


15


VCC


16


R1IN


13


R2IN


8


R2OUT


9


C1+


1


C1-


3


C2+


4


C2-


5


V+


2


V-


6


U2


ICL232




Title

Size Document Number Rev

Date: Sheet of

<Doc> <RevCode>

<Title>

E

1 1 Saturday, April 05, 2008

RE0/RD/AN5

8

RE1/WR/AN6

9

RE2/CS/AN7

10

GND

12

OSC2/CLKOUT

14

RC0/T1OSO/T1CKI

15

RC1/T1OSI/CCP2

16

RC2/CCP1

17

RC3/SCK/SCL

18

RD0/PSP0

19

RD1/PSP1

20

RD2/PSP2

21

RD3/PSP3

22

RC4/SDI/SDA

23

RC5/SDO

24

RC6/TX/CK

25

RC7/RX/DT

26

RD4/PSP4

27

RD5/PSP5

28

RD6/PSP6

29

RD7/PSP7

30

GND

31

MCLR/VPP

1

OSC1/CLKIN

13

RA0/AN0

2

RA1/AN1

3

RA2/AN2

4

RA3/AN3/VREF

5

RA4/TOCKI

6

RA5/AN4/SS

7

RBO/INT

33

RB1

34

RB2

35

RB3

36

RB4

37

RB5

38

RB6

39

RB7

40

VDD

11

VDD

32

U3

PIC16f877

SW1

SERIAL SOCKET

C4

0.1u

1 2

J1

Input Voltage

S8

Program

X1

4MHz

D1

1N4007

C8

12p

C9

12p

R1

33K

C12

6.8u

S9

RESET

C11

0.1u

C10

0.1u

D2

LED

C3

1u

C5

1u

C6

1u

VIN

1

GND

2

VOUT

3

U1Regulator

C7

1u

5V

C1

10u

C2

0.1u

R1OUT

12

T1OUT

14

T2OUT

7

T2IN

10

T1IN

11

GND

15

VCC

16

R1IN

13

R2IN

8

R2OUT

9

C1+

1

C1-

3

C2+

4

C2-

5

V+

2

V-

6

U2

ICL232


Then I test functionality to basic circuit and it work good.

(2) interface two pic to the PC
Connect the Two pic max 232 throw a third max 232 to interface both pic to the serial port DB9 in the PC the following schematic to the connection .
[image: image9.emf][image: image3.emf]VCC


VCC


RC7


RC6


C39


1u


C40


0.1u


C41


1u


C42


1u


C43


1u


C44


1u


R1OUT


12


T1OUT


14


T2OUT


7


T2IN


10


T1IN


11


GND


15


VCC


16


R1IN


13


R2IN


8


R2OUT


9


C1+


1


C1-


3


C2+


4


C2-


5


V+


2


V-


6


U12


ICL232


RC7


C46


0.1u


C47


1u


5


9


4


8


3


7


2


6


1


P2


CONNECTOR DB9


1


2


3


U13A


7408


C48


0.1u


R1OUT


12


T1OUT


14


T2OUT


7


T2IN


10


T1IN


11


GND


15


VCC


16


R1IN


13


R2IN


8


R2OUT


9


C1+


1


C1-


3


C2+


4


C2-


5


V+


2


V-


6


U14


ICL232


C49


1u


1


2


7407


C50


1u


C51


1u


3


4


7407


C52


1u


C53


1u


C54


1u


R1OUT


12


T1OUT


14


T2OUT


7


T2IN


10


T1IN


11


GND


15


VCC


16


R1IN


13


R2IN


8


R2OUT


9


C1+


1


C1-


3


C2+


4


C2-


5


V+


2


V-


6


U16


ICL232


0


0


RC6


0




VCC

VCC

RC7

RC6

C39

1u

C40

0.1u

C41

1u

C42

1u

C43

1u

C44

1u

R1OUT

12

T1OUT

14

T2OUT

7

T2IN

10

T1IN

11

GND

15

VCC

16

R1IN

13

R2IN

8

R2OUT

9

C1+

1

C1-

3

C2+

4

C2-

5

V+

2

V-

6

U12

ICL232

RC7

C46

0.1u

C47

1u

5

9

4

8

3

7

2

6

1

P2

CONNECTOR DB9

1

2

3

U13A

7408

C48

0.1u

R1OUT

12

T1OUT

14

T2OUT

7

T2IN

10

T1IN

11

GND

15

VCC

16

R1IN

13

R2IN

8

R2OUT

9

C1+

1

C1-

3

C2+

4

C2-

5

V+

2

V-

6

U14

ICL232

C49

1u

1 2

7407

C50

1u

C51

1u

3 4

7407

C52

1u

C53

1u

C54

1u

R1OUT

12

T1OUT

14

T2OUT

7

T2IN

10

T1IN

11

GND

15

VCC

16

R1IN

13

R2IN

8

R2OUT

9

C1+

1

C1-

3

C2+

4

C2-

5

V+

2

V-

6

U16

ICL232

0

0

RC6

0

.

(3) Connect DS1302 to Pic.
DS1302  is Real-Time Clock  counts Seconds, Minutes, Hours, Date of the Month, Month, Day of the Week, and Year with Leap-Year Compensation Valid Up to 2100 , it contain the following major component :- 
· 31 x 8 RAM for Scratchpad Data Storage .
· Serial I/O for Minimum Pin Count
· 2.0V to 5.5V Full Operation
· 2.0V to 5.5V Full Operation
· Uses less than 300nA at 2.0V .

· Simple 3-wires interface .

· TTL compatible (VCC =5V).

· [image: image10.emf]Single Byte or multiple Byte (Burst mode)Data transfer for read or write of clock or RAM Data.
[image: image11.emf]
Interfacing the DS1302 with a microcontroller  is simplified by using synchronous serial communication. Only three wires are required to communicate with the clock/RAM: CE, I/O (data line), and SCLK (serial clock). Data can be transferred to and from the clock/RAM 1 byte at a time or in a burst of up to 31 bytes. The DS1302 is designed to operate on very low power and retain data and clock information on less than 1μW. the following schematic to interfacing  DS1302 to the PIC16F877.
[image: image4.emf]VCC


RB3


RB2


RB1


68u


0.1u


4


X1


2


X2


3


RST


5


I/O


6


SCLK


7


VCC


1


VCC


8


U12


DS1302


32.768kHz


0




VCC

RB3

RB2

RB1

68u

0.1u

4

X1

2

X2

3

RST

5

I/O

6

SCLK

7

VCC

1

VCC

8

U12

DS1302

32.768kHz

0


(4) Building an infrared  transmitter and receiver circuit :
To detect intruder when enter the door ,I build an infrared circuit ,where the transmitter circuit contain of an infrared transmitter led with 555 timer to generate 38khz infrared light as shown in the following schematic circuit.
[image: image12.jpg]serial Port#

s

o opened.

Suian 1] B3

Yioden

i

-

s

s

-

Oulding A

n

rom Mont} e

Buniding

. Get Intruder Log ]
]

06-May-08 2:31 ¢



[image: image5.emf]infrared


10k


4.7k


22k


1nf


OUT


3


DISCH


7


VCC


8


TRIG


2


RESET


4


CONT


5


THRES


6


U1


ZSCT1555


VCC_BAR


3


1


2


Q2A


BC548


10nf


C


R2


R1




infrared

10k

4.7k

22k

1nf

OUT

3

DISCH

7

VCC

8

TRIG

2

RESET

4

CONT

5

THRES

6

U1

ZSCT1555

VCC_BAR

3

1

2 Q2A

BC548

10nf

C

R2 R1


[image: image13.jpg]d

.5'8110~|Jl|0\
. e 80 L N L )
TTYOT OO TR > o .
8 29 8 ¢80 & aam o0 ow ® % o » « o
O % 9 s eV 00 o800 o "« e 8
LR B 2 P & & 9 » & 9 8 v « % @
& S A 9 e ap oo o « a a
- a9 00 e & = w @
[ e s ofi] e 40 a
. " e o ofi] -WH-:.
L} « » » s e of] ,-QM..Q....
f} & & & 8 5 & 0 0 o of] aof:.,ann
S s s o908 00 o] DEE T
S 8800000 e T[] °0 h -
S 8 5 8 % 2 n e ) 9 °0 > o
nccoavsno. ﬁ. °I » °
LI L B (41 —Ul! .
. s - a OC‘ Ld
Ll G []® ¢ » -
" e G fy}e o « °
' ® ] » *
L -.UI..
. P
j . o
L3 - -
«u-aooolc. m
S e e P e &
LR B N B N B s 2«
.. - -\.o
. . L o w
.. - -n .o
. - e «n e
LI Y - e 0 "0 e
.- . N I A
.' - - 5 2 5 " "2 o0
'( - LR B B B B N B B B
- . % 2 8 " e " e e
- LR B B N B B

c

aﬁ‘
2
*L
o
o

s 0 Y EEREEREEEEEE S
EEEEEREEREEEFEERES & & 0

MEIEEEEEREEEREEERERRER & &

o » & N EEEEEE

5 a8 o 8 a " 4 . " . . o

N EEEREEERERFERES =

WEEEREEEERERS &8 0

o I EEEEEESE R RS S

> p o
",
&
]
i,
s u
> 2
I
‘. 2
N
>
L
s 2
4

b. ¢
¥ ¢
&
» o b o
\‘\0 -
s
f cl
/ Qx’

‘

*

.nnoa’

Y asaSas

.
-

>

»

.

.

-

- n

a "

- TR

-9 > 2+ .

s e » * 9 % 2

LR I 2 9 % 9

an e 522 4 e

e * * s =
CE - . >

06-May-08 2:27 ¢



In the other side there an infrared receiver TSOP1738 which only detect light of 38Khz frequency .
[image: image14.jpg]v o 000000000 O INEE_
parstee o -9 3 N4 .y.g..--.nua. - s awE XLt LR R R B
L I = . » .- - - . .
s ensenn n-:. S AR AN SRR BSRIN - epmgeapeapee € = ¥ L R R B B
chnesie 4404+ i sy e SR e B ~AS YREE &% sa N AL
Sraenen -n...-,» e T R R P assapsOeUEDSS h
THERacee EERE T e LR - S ARG ECINAEANNSES
b i e N el - D SRR . saasRTEIGNONTANS
s hosRES e s g - LesasReEERRERISRNTS
N sedee e g " - & . AR RREHELERERRESD
. LR pU— ™ . ... J.nnl.o"lio‘n
- ' R 'EEREE - - LB R R T B BCR e S e i
Sesrrns S e 4 - . “anea accuv O S
. LR ceanees - - - a . » AN BB OBENS SN
. seenyn EE L LR ... -~ w PRI
. $enes s s nna L a4 ans R O —
. e an ananss DY aApsanan anB ans0o e
sesssses PO - sdse - e B s wn @ BN \
el by 1 . &a» 7 @ E Erianat:
ss esaen R’ awan aa eroe s am s e s ‘,
. L TR RN . - » g B~ c4aann e B
. tr e LI R -~ - » ARNE T s assassnsy
s samne s de oW - P PAas e an AN A n SRR SRR -4
sas e s o o alile - P . ol » ~ o Sitais AR KA Y E e
- L ] » s Ny . .enN I N R R RN E N EERERERE
. R R L " S AAAARANSIAANANCusnpa sy R
sasavae SRS . O AT aa e e oA NETE IR
» ' R E R Ly =0 . T o RO B B aas AN L
TR AL RN, CIR - D I N 5 aa AP o 4 N
casanas a o'k yan e . Ea a B o E Ay
R s 85 Ny PR . . N et -1 ]
. I PR B W LR B - - s ‘cv- \A )
R ) L s ane IBmAPEEESESS A PR - 4
0  E R RN L A T e .-.“ -~ S & &

& 280 @ e e ¥a vi.voonCGD L] LI R .

T TR R RN EENER NS LR - P, s
-o.-on-vooo-tunt-n-u.prnu\.co-.o-nnosup-lnln:n v R Y Ak « a5 &

R E b ssssessssssccccconsosce UEREEIEFRERESEESERSS e 5 7 s Al o

{ /
! /

|\ 06-May-08 2:26 ¢



[image: image6.emf]LED


1


3


2


1u


VCC_BAR


TSOP1738




LED

1 3 2

1u

VCC_BAR

TSOP1738


(5) Connect the infrared receiver circuit to the PIC 16F877
      in this step I connect an infrared receiver output pin(pin_3) to the PORT_A in  the pic when someone enter the door ,the port_A pin become high so it inform the pc the action done.

Also I connect an switch to the PORT_A to indicate the some door or window are opened. So when the window is opened

The pic will detect this action and send the time ,date and place of the intruder.

The following  schematic to the connection to all project.

[image: image7.emf]2


1


Title


Size


Document Number


Rev


Date:


Sheet


of


<Doc>


<RevCode>


<Title>


E


1


1


Monday, May 05, 2008


3


1u


C27


1u


TSOP1738


RE0/RD/AN5


8


RE1/WR/AN6


9


RE2/CS/AN7


10


GND


12


OSC2/CLKOUT


14


RC0/T1OSO/T1CKI


15


RC1/T1OSI/CCP2


16


RC2/CCP1


17


RC3/SCK/SCL


18


RD0/PSP0


19


RD1/PSP1


20


RD2/PSP2


21


RD3/PSP3


22


RC4/SDI/SDA


23


RC5/SDO


24


RC6/TX/CK


25


RC7/RX/DT


26


RD4/PSP4


27


RD5/PSP5


28


RD6/PSP6


29


RD7/PSP7


30


GND


31


MCLR/VPP


1


OSC1/CLKIN


13


RA0/AN0


2


RA1/AN1


3


RA2/AN2


4


RA3/AN3/VREF


5


RA4/TOCKI


6


RA5/AN4/SS


7


RBO/INT


33


RB1


34


RB2


35


RB3


36


RB4


37


RB5


38


RB6


39


RB7


40


VDD


11


VDD


32


U3


PIC16f877


VCC


13


11


9


7


RE0/RD/AN5


8


RE1/WR/AN6


9


RE2/CS/AN7


10


GND


12


OSC2/CLKOUT


14


RC0/T1OSO/T1CKI


15


RC1/T1OSI/CCP2


16


RC2/CCP1


17


RC3/SCK/SCL


18


RD0/PSP0


19


RD1/PSP1


20


RD2/PSP2


21


RD3/PSP3


22


RC4/SDI/SDA


23


RC5/SDO


24


RC6/TX/CK


25


RC7/RX/DT


26


RD4/PSP4


27


RD5/PSP5


28


RD6/PSP6


29


RD7/PSP7


30


GND


31


MCLR/VPP


1


OSC1/CLKIN


13


RA0/AN0


2


RA1/AN1


3


RA2/AN2


4


RA3/AN3/VREF


5


RA4/TOCKI


6


RA5/AN4/SS


7


RBO/INT


33


RB1


34


RB2


35


RB3


36


RB4


37


RB5


38


RB6


39


RB7


40


VDD


11


VDD


32


U4


PIC16f877


C13


0.1u


1


2


J2


Input Voltage


1


2


J4


GND


S10


Program


X2


4MHz


D3


1N4007


C14


12p


C15


12p


R2


33K


12


C16


6.8u


6


8


10


S11


RESET


C17


0.1u


C18


0.1u


D4


LED


C19


1u


C20


1u


C21


1u


VIN


1


GND


2


VOUT


3


U5


Regulator


C22


1u


5V


C23


10u


C24


0.1u


infrared receiver


LED


infrared receiver


1


3


2


R1OUT


12


T1OUT


14


T2OUT


7


T2IN


10


T1IN


11


GND


15


VCC


16


R1IN


13


R2IN


8


R2OUT


9


C1+


1


C1-


3


C2+


4


C2-


5


V+


2


V-


6


U6


ICL232


1u


TSOP1738


68u


1


3


2


W1


0.1u


1


3


2


W2


C4


0.1u


1


3


2


W3


1


2


J1


Input Voltage


1


2


J3


GND


VCC


C28


1u


4


S8


Program


X1


4MHz


D1


1N4007


C8


12p


C9


12p


VCC


X1


2


X2


3


RST


5


I/O


6


SCLK


7


VCC


1


VCC


8


U10


DS1302


5


9


4


8


3


7


2


6


1


P1


CONNECTOR DB9


R1


33K


C12


6.8u


S9


RESET


1


2


3


U8A


7408


C11


0.1u


C10


0.1u


C29


0.1u


1


2


BUZZER


D2


LED


R1OUT


12


T1OUT


14


T2OUT


7


T2IN


10


T1IN


11


GND


15


VCC


16


R1IN


13


R2IN


8


R2OUT


9


C1+


1


C1-


3


C2+


4


C2-


5


V+


2


V-


6


U7


ICL232


C3


1u


1


2


7407


C25


1u


C5


1u


3


4


7407


C6


1u


VIN


1


GND


2


VOUT


3


U1


Regulator


C7


1u


32.768kHz


5V


C1


10u


C2


0.1u


C26


1u


R3


1K


VCC


1


2


BUZZER


R4


1K


1


3


2


W4


1


3


2


W5


1


3


2


W6


R5


1K


R6


1K


R7


1K


R8


1K


LED


1


3


2


1u


TSOP1738


R1OUT


12


T1OUT


14


T2OUT


7


T2IN


10


T1IN


11


GND


15


VCC


16


R1IN


13


R2IN


8


R2OUT


9


C1+


1


C1-


3


C2+


4


C2-


5


V+


2


V-


6


U2


ICL232


1


14


2


2


LCD


LED


1


3


2


1u


TSOP1738


68u


0.1u


4


X1


2


X2


3


RST


5


I/O


6


SCLK


7


VCC


1


VCC


8


U11


DS1302


32.768kHz


5


4


3


LED




2 1

Title

Size Document Number Rev

Date: Sheet of

<Doc> <RevCode>

<Title>

E

1 1 Monday, May 05, 2008

3

1u

C27

1u

TSOP1738

RE0/RD/AN5

8

RE1/WR/AN6

9

RE2/CS/AN7

10

GND

12

OSC2/CLKOUT

14

RC0/T1OSO/T1CKI

15

RC1/T1OSI/CCP2

16

RC2/CCP1

17

RC3/SCK/SCL

18

RD0/PSP0

19

RD1/PSP1

20

RD2/PSP2

21

RD3/PSP3

22

RC4/SDI/SDA

23

RC5/SDO

24

RC6/TX/CK

25

RC7/RX/DT

26

RD4/PSP4

27

RD5/PSP5

28

RD6/PSP6

29

RD7/PSP7

30

GND

31

MCLR/VPP

1

OSC1/CLKIN

13

RA0/AN0

2

RA1/AN1

3

RA2/AN2

4

RA3/AN3/VREF

5

RA4/TOCKI

6

RA5/AN4/SS

7

RBO/INT

33

RB1

34

RB2

35

RB3

36

RB4

37

RB5

38

RB6

39

RB7

40

VDD

11

VDD

32

U3

PIC16f877

VCC

13

11

9

7

RE0/RD/AN5

8

RE1/WR/AN6

9

RE2/CS/AN7

10

GND

12

OSC2/CLKOUT

14

RC0/T1OSO/T1CKI

15

RC1/T1OSI/CCP2

16

RC2/CCP1

17

RC3/SCK/SCL

18

RD0/PSP0

19

RD1/PSP1

20

RD2/PSP2

21

RD3/PSP3

22

RC4/SDI/SDA

23

RC5/SDO

24

RC6/TX/CK

25

RC7/RX/DT

26

RD4/PSP4

27

RD5/PSP5

28

RD6/PSP6

29

RD7/PSP7

30

GND

31

MCLR/VPP

1

OSC1/CLKIN

13

RA0/AN0

2

RA1/AN1

3

RA2/AN2

4

RA3/AN3/VREF

5

RA4/TOCKI

6

RA5/AN4/SS

7

RBO/INT

33

RB1

34

RB2

35

RB3

36

RB4

37

RB5

38

RB6

39

RB7

40

VDD

11

VDD

32

U4

PIC16f877

C13

0.1u

1 2

J2

Input Voltage

1 2

J4

GND

S10

Program

X2

4MHz

D3

1N4007

C14

12p

C15

12p

R2

33K

12

C16

6.8u

6

8

10

S11

RESET

C17

0.1u

C18

0.1u

D4

LED

C19

1u

C20

1u

C21

1u

VIN

1

GND

2

VOUT

3

U5Regulator

C22

1u

5V

C23

10u

C24

0.1u

infrared receiver

LED

infrared receiver

1 3 2

R1OUT

12

T1OUT

14

T2OUT

7

T2IN

10

T1IN

11

GND

15

VCC

16

R1IN

13

R2IN

8

R2OUT

9

C1+

1

C1-

3

C2+

4

C2-

5

V+

2

V-

6

U6

ICL232

1u

TSOP1738

68u

1

3

2

W1

0.1u

1

3

2

W2

C4

0.1u

1

3

2

W3

1 2

J1

Input Voltage

1 2

J3

GND

VCC

C28

1u

4

S8

Program

X1

4MHz

D1

1N4007

C8

12p

C9

12p

VCC

X1

2

X2

3

RST

5

I/O

6

SCLK

7

VCC

1

VCC

8

U10

DS1302

5

9

4

8

3

7

2

6

1

P1

CONNECTOR DB9

R1

33K

C12

6.8u

S9

RESET

1

2

3

U8A

7408 C11

0.1u

C10

0.1u

C29

0.1u

1

2

BUZZER

D2

LED

R1OUT

12

T1OUT

14

T2OUT

7

T2IN

10

T1IN

11

GND

15

VCC

16

R1IN

13

R2IN

8

R2OUT

9

C1+

1

C1-

3

C2+

4

C2-

5

V+

2

V-

6

U7

ICL232

C3

1u

1 2

7407

C25

1u

C5

1u

3 4

7407

C6

1u

VIN

1

GND

2

VOUT

3

U1Regulator

C7

1u

32.768kHz

5V

C1

10u

C2

0.1u

C26

1u

R3

1K

VCC

1

2

BUZZER

R4

1K

1

3

2

W4

1

3

2

W5

1

3

2

W6

R5

1K

R6

1K

R7

1K

R8

1K

LED

1 3 2

1u

TSOP1738

R1OUT

12

T1OUT

14

T2OUT

7

T2IN

10

T1IN

11

GND

15

VCC

16

R1IN

13

R2IN

8

R2OUT

9

C1+

1

C1-

3

C2+

4

C2-

5

V+

2

V-

6

U2

ICL232

1

14

22

LCD

LED

1 3 2

1u

TSOP1738

68u

0.1u

4

X1

2

X2

3

RST

5

I/O

6

SCLK

7

VCC

1

VCC

8

U11

DS1302

32.768kHz

5

4 3

LED


(6)Programming the PIC 16F877 :
By writing a program using PIC C program .in the code I start initializing the RTC(real time clock) DS1302 by setting it time and date .

The PIC start work   at specific time and also stop at specific time ,where it start polling to the port A if such an action done  directly send message to the PC to inform to the place of intruder

As shown in the sample code below:-
[image: image15.bmp][image: image16.bmp]
[image: image17.bmp]
[image: image18.bmp]
(7) Building an interface application in C#  

In the C# application program read the incoming  message  from the serial port and compare it with the expected message if match indict in the building picture to the place and kind of action done(window open , entering from door) ,

Also the application automatically set up call through modem using AT command when an warning happened, and when the  call answered  play a recorded sound to inform the place, and kind of warning.

Also application have a database to store the all action done ,time ,date and place  where the user able to retrieve that log file.
The following shown some sample ofm the code:
[image: image19.bmp]
       



(8)Completion of Project :

After finishing all of the project connection and programming

I make  checking to it functionality as shown in the following picture below.  


[image: image8.png]



Basic circuit Schematic





Interface Two Max to PC Serial   PO///////////////////////port





�





�





Infrared  Transmitter Circuit





Infrared receiver circuit 





�





if (rcvdata == "A0")


            {


                if (tt.Enabled == false)


                {


                    cmd.CommandText = "insert into LOG Values('" + d1 + "','" + t1 + "','window opened','A','1')";


                   // obj.BeginInvoke (WriteToTextFieldMethod ,"vvbnnnnnnnnnnnnnnnnnn");


                    this.Invoke(obj, "window opened");





                   // wav = 0;


                    cmd.ExecuteNonQuery();


                    DrawImage_1(310, 300);


                   


                    tt.Start();


                    tt.Enabled = true;


                    if(busy == false)


                    {


                        wav = 0;


                        Execute();


                    }


                    else


                    {


                        waited[0] = 5;


                        index = index + 1;


                    }


                }


             // MessageBox.Show("A0");


                //t.Enabled = true;


            }


            else if (rcvdata == "A1")


            {


                if (a1.Enabled == false)


                {  


                    


                    //MessageBox.Show(t1);


                    //MessageBox.Show(d1);


                   // wav = 1;


                    cmd.CommandText = "insert into LOG Values('" + d1+ "','" + t1 + "','window opened','A','2')";


                   this.Invoke(obj1,"window opened");


                    cmd.ExecuteNonQuery();


                    DrawImage_1(310, 260);


                    a1.Start();


                   


                    a1.Enabled = true;


                    if (busy == false)


                    {


                        wav = 1;


                        Execute();


                    }


                    else


                    {


                        waited[1] = 5;


                        index = index + 1;


                           


                    }


                }


                //MessageBox.Show("A1");


            }


            else if (rcvdata == "A2")


            {


                if (a2.Enabled == false)


                {


                    //string tt = serialPort1.ReadLine();


                    //string dd = serialPort1.ReadLine();


                   //MessageBox.Show(tt);


                   //MessageBox.Show(dd);


                    cmd.CommandText = "insert into LOG Values('" + d1 + "','" +t1+ "','window opened','A','3')";


                    this.Invoke(obj2, "window opened");


                    cmd.ExecuteNonQuery();


                    DrawImage_1(310, 220);


                    a2.Start();


                    a2.Enabled = true;


                }


                //MessageBox.Show("A2");


            }


            else if (rcvdata == "A3")


            {


                if (a3.Enabled == false)


                {


                    this.Invoke(obj3, "door entered opened");


                    cmd.CommandText = "insert into LOG Values('" + d1 + "','" + t1+ "','door entered','A','1')";


                    DrawImage_1(310, 170);


                    a3.Start();


                    a3.Enabled = true;


                }


                //MessageBox.Show("A3");


            }





            else if (rcvdata == "B0")


            {


               // MessageBox.Show("b0");


                if (b0.Enabled == false)


                {


                    this.Invoke(obj4, "window opened");


                    cmd.CommandText = "insert into LOG Values('" + d1 + "','" + t1 + "','window opened','B','1')";


                   //MessageBox.Show( "window open in building B Flat 1");


                    TextBox t = new TextBox();


                   // t.Location.X = 456;


                    //t.Location.Y = 286;


                    t.Text = "hi";


                    //wav = 3;


                    cmd.ExecuteNonQuery();


                    DrawImage_1(60, 300);


                    b0.Start();


                    b0.Enabled = true;


                    if (wav == 7)


                    {


                        wav = 3;


                        Execute();


                    }


                }


               // MessageBox.Show("b0");


            }





            else if (rcvdata == "B1")


            {


                if (b1.Enabled == false)


                {


                    cmd.CommandText = "insert into LOG Values('" + d1 + "','" + t1 + "','window opened','B','2')";


                    this.Invoke(obj5, "window opened");


                    cmd.ExecuteNonQuery();


                    DrawImage_1(60, 260);


                    //wav = 4;


                    b1.Start();


                    b1.Enabled = true;


                    if (wav == 7)


                    {


                        wav = 4;


                        Execute();


                    }


                }


                //MessageBox.Show("A3");


            }





            else if (rcvdata == "B2")


            {


                if (b2.Enabled == false)


                {


                    cmd.CommandText = "insert into LOG Values('" + d1 + "','" + t1 + "','window opened','B','3')";


                    this.Invoke(obj6, "window opened");


                    cmd.ExecuteNonQuery();


                    DrawImage_1(60, 220);


                    


                    b2.Start();


                    b2.Enabled = true;


                }


                //MessageBox.Show("A3");


            }





            else if (rcvdata == "B3")


            {


                if (b3.Enabled == false)


                {


                    cmd.CommandText = "insert into LOG Values('" + d1 + "','" + t1 + "','door entered','B','1')";


                    this.Invoke(obj7, "door entered");


                    DrawImage_1(60, 170);


                    b3.Start();


                    b3.Enabled = true;


                }


                //MessageBox.Show("A3");


            }





Serial Port receive function





 SendCommand("ATZ");


GetResponse(1, true);


SendCommand("AT+FCLASS=8");


GetResponse(1, true);


SendCommand("AT+VSM=128,8000");


GetResponse(1, true);


 SendCommand("ATDT" + m_phoneNumber);


GetResponse(60, false);


//SendCommand("ATH0");


 SendCommand("AT+VTX");


System.Threading.Thread.Sleep(500);


SendVoiceData();





if (wav == 0)


            {


                if (m_whdr == null) return;


                dataLength = m_whdr.data.Length;





                if (dataLength == 0) return;





                try


                {


                    // MessageBox.Show("Start sending WAVE data...");


                    while (true)


                    {


                        serialPort2.Write(m_whdr.data, offset, blockSize);


                        if (blockSize < 1024) break;


                        offset += blockSize;


                        if (dataLength - offset < 1024) blockSize = dataLength - offset;


                    }


                    //MessageBox.Show("Done sending WAVE data.");





                }


                catch


                {


                    MessageBox.Show("Error sending WAVE data.");


                }


            }


            else if (wav == 1)


            {


                if (m_whdr1 == null) return;


                dataLength = m_whdr1.data.Length;


                if (dataLength == 0) return;


                try


                {


                    while (true)


                    {


                        serialPort2.Write(m_whdr1.data, offset, blockSize);


                        if (blockSize < 1024) break;


                        offset += blockSize;


                        if (dataLength - offset < 1024)


                            blockSize = dataLength - offset;





                    }


                }


                catch


                {


                    MessageBox.Show("Error sending WAVE data.");


                }


            }


            else if (wav == 2)




















            {


                if (m_whdr2 == null) return;


                dataLength = m_whdr2.data.Length;


                if (dataLength == 0) return;


                try


                {


                    while (true)


                    {


                        serialPort2.Write(m_whdr2.data, offset, blockSize);


                        if (blockSize < 1024) break;


                        offset += blockSize;


                        if (dataLength - offset < 1024)


                            blockSize = dataLength - offset;





                    }


                }





Set up call through modem





myPen.Color = Color.Red;


            myPen.DashStyle = System.Drawing.Drawing2D.DashStyle.Solid;


            Image img = Image.FromFile(@"C:\Documents and Settings\Administrator\My Documents\Visual Studio 2005\Projects\test\test\alarm.gif");


            if (x == 0 && y == 0)


            {


                if ((tt.Enabled == false) || (a1.Enabled == false) || (a2.Enabled == false) || (a3.Enabled == false)||(b0.Enabled == false) || (b1.Enabled == false) || (b2.Enabled == false) || (b3.Enabled == false))


                {


                    myPen.Color = Color.White;


                    for (int f = 0; f < this.Width; f++)


                    {


                        oGraphics.DrawLine(myPen, f, 0, f, this.Height);


                    }


                }


                


            }


            else


            {


                myPen.Color = Color.White;


                oGraphics.DrawImage(img, x, y);


            }





            if (tt.Enabled == true)


            {


               oGraphics.DrawImage(img, 310, 300);


            }


            if (a1.Enabled == true)


            {


                oGraphics.DrawImage(img, 310, 260);


            }


            if (a2.Enabled == true)


            {


                oGraphics.DrawImage(img, 310, 220);


            }


            if (a3.Enabled == true)


            {


                oGraphics.DrawImage(img, 310, 170);


            }


            if (b0.Enabled == true)


            {


                oGraphics.DrawImage(img, 60, 300);


            }


            if (b1.Enabled == true)


            {


                oGraphics.DrawImage(img, 60, 260);


            }


            if (b2.Enabled == true)


            {


                oGraphics.DrawImage(img, 60, 220);


            }


            if (b3.Enabled == true)


            {


                oGraphics.DrawImage(img, 60, 170);


            }





Drawing function





void set_clock(){


   BYTE day, mth, year, dow, hour, min;





   printf("\r\nPress ENTER after 1 digit answers.");


    restart_wdt();


   printf("\r\nYear 20: ");


    restart_wdt();


   year=get_number();


   printf("\r\nMonth: ");


    restart_wdt();


   mth=get_number();


   printf("\r\nDay: ");


    restart_wdt();


   day=get_number();


   printf("\r\nWeekday 1-7: ");


   dow=get_number();


   printf("\r\nHour: ");


    restart_wdt();


   hour=get_number();


   printf("\r\nMin: ");


    restart_wdt();


   min=get_number();





   rtc_set_datetime(day,mth,year,dow,hour,min);


   printf("\r\n\n");


     restart_wdt();





}





while (1) {


restart_wdt();


      rtc_get_date( day, mth, year, dow);


      rtc_get_time( hour, min, sec );


//restart_wdt();


 printf(LCD_PUTC,"\f");


 restart_wdt();


  printf(lcd_putc,"%02u/%02u/%02u  \n",hour,min,sec);


  restart_wdt();


 printf(lcd_putc,"%02u/%02u/%02u  \n",mth,day,year);


      delay_ms(250);


      restart_wdt();


if((hour==8)&&(min>=0)&&(sec>=0)&&(min<60)&&(sec<=60))


      {


        data = input_a();


       if( bit_test(data,0))


       {


       printf("B0\n");


  printf("%02u/%02u/%02u  \n",hour,min,sec);


printf(,"%02u/%02u/%02u  \n",mth,day,year);


       restart_wdt();


      //  output_high(pin_d2);


       }


       if( bit_test(data,1))


       {


       printf("B1");


       restart_wdt();


       //output_high(pin_d2);


       }





Setting RTC


time





Polling port_A





�





�





�








