Mohamed Khaled Mousa Ali – Reg. No:11642774

Mohannad Izzat abed Al-Qadir Mousa – Reg. No: 11744278

Mohammed Nasser Haroon Kayed – Reg. No: 11524055

Faten Saleh Ahmad Daraghmeh – Reg. No:11717844

Economic & Environmental Impact assessment for porous asphalt applications in Nablus, Palestine

Nablus city faces many problems related to water especially in the winter, these problems include flooding, accidents, and hassling the pedestrians. In addition to that, ground water is limited, runoff flow is being wasted, and the citizens of the city don’t have access to clean water all the time. This study aimed to tackle and solve these problems, the solution to solve them was to somehow increase the accessed ground water while reducing the surface runoff, this was done by using a permeable paving system, more specifically, the porous asphalt. To achieve this, determining the appropriate streets for using porous asphalt use done, then a porous asphalt design was made and a feasibility study was done to see whether the economic and environmental aspects for porous asphalt add any benefits or harms compared to the conventional asphalt design.

The study introduced a new and important scientific contribution, since it is the first time in Palestine that the implementation of porous asphalt was investigated. The economic aspect including the cost of construction, and the cost of saved underground water were determined. Finally, after analyzing all the results the following findings were concluded:

• The study concluded that the most suitable permeable pavement type to be implemented is the full-infiltration porous asphalt design.

• Porous asphalt is applicable in the city of Nablus, especially in areas where:

1. The soil type has to be marl soil with stones.

2. The streets have to be classified as local streets with speed limit up to 50 km/hr, and have a low traffic volume.

3. The streets have to have a grade less than 5%.

• The best suitable design for porous asphalt layers was as follows: a porous asphalt layer with a thickness of 5cm, an asphalt-treated permeable base layer with a thickness of 8cm, a reservoir course layer with a thickness of 40cm, and the last layer is the natural uncompacted soil layer as subgrade because it has good permeability properties, other possible layers include: choker course layer, filter course layer and geotextiles, they were not used.

• The application of porous asphalt in the selected streets in the city of Nablus as an alternative to conventional asphalt has important economic benefits, as it saves about of 1,912,000 ILS for the return period of the study which is 20 years. This saving includes the initial construction cost (material costs and water drainage system costs), and the future maintenance and operating costs (maintenance cost for the streets, maintenance cost for the water drainage system and the cost for acquiring, operating and maintenance of the cleaning machine).

• The application of porous asphalt saves the cost of construction and maintenance of the water drainage system, since it allows stormwater runoff to filter through the voids. However, porous asphalt must be vacuumed periodically twice a year to keep the pours opened, this leads to an increase in the maintenance cost.

• The results from the Environmental impact assessment concluded that the groundwater is recharged at a rate of 107,404.7 m3 /year. This amount of water can satisfy that daily capita needs in Nablus city for 7 days while generating upwards of 37,023,350 ILS per year for the government. • The use of porous asphalt in Nablus has many environmental benefits:

1. Improving the water quality by filtering and decreasing pollutants.

2. Reduce the water runoff causing a reduction of thermal pollution compared to conventional asphalt.

3. Contribute to solving the flooding problem in Nablus.

4. Contribute to reducing the accidents rate.

5. Filtering the rainwater leads to reduce the cost and work load imposed upon water filtering plants in the winter.

تقييم الأثر الاقتصادي والبيئي لتطبيقات الأسفلت المسامي في مدينة نابلس، فلسطين

تواجه مدينة نابلس العديد من المشاكل المتعلقة بالمياه خاصة في فصل الشتاء، وتشمل هذه المشاكل الفيضانات والحوادث وإزعاج المشاة. بالإضافة إلى ذلك، فإن المياه الجوفية محدودة، وتدفق الجريان السطحي يضيع سدى، ولا يستطيع سكان المدينة الوصول إلى المياه النظيفة طوال الوقت. هدفت هذه الدراسة إلى معالجة هذه المشكلات وحلها، وكان الحل هو زيادة المياه الجوفية التي يتم الوصول إليها بطريقة ما مع تقليل الجريان السطحي، وقد تم ذلك باستخدام نظام رصف قابل للاختراق، وبشكل أكثر تحديدًا، الإسفلت المسامي. ولتحقيق ذلك، تم تحديد الشوارع المناسبة لاستخدام الإسفلت المسامي، ثم تم عمل تصميم أسفلت مسامي، وتم عمل دراسة جدوى لمعرفة ما إذا كانت الجوانب الاقتصادية والبيئية للإسفلت المسامي تضيف أي فوائد أو أضرار مقارنة بتصميم الأسفلت التقليدي.

قدمت الدراسة مساهمة علمية جديدة وهامة، حيث إنها المرة الأولى في فلسطين التي يتم فيها التحقيق في استخدام الأسفلت المسامي. تم تحديد الجانب الاقتصادي بما في ذلك تكلفة البناء، وتكلفة توفير المياه الجوفية. وأخيرًا، بعد تحليل جميع النتائج تم التوصل إلى النتائج التالية:

• توصلت الدراسة إلى أن نوع الرصف الأكثر نفاذاً والمطلوب تنفيذه هو الإسفلت المسامي كامل الارتشاح.

• الإسفلت المسامي قابل للتطبيق في مدينة نابلس، وخاصة في المناطق التي:

1. نوع التربة يجب أن يكون مرل بالحجارة.

2. يجب تصنيف الشوارع على أنها شوارع محلية بحد أقصى لسرعة تصل إلى 50 كم / ساعة ، وذات كثافة مرورية منخفضة.

3. أن تكون ميول الشوارع أقل من 5٪.

• كان أفضل تصميم مناسب للطبقات الإسفلتية المسامية كما يلي: طبقة إسفلتية مسامية بسمك 5 سم، وطبقة قاعدة نفاذة ومعالجة بالإسفلت بسمك 8 سم، وطبقة خزان بسمك 40 سم، وطبقة أخيرة هي طبقة التربة الطبيعية غير المضغوطة كطبقة سفلية لأنها تتمتع بخصائص نفاذية جيدة، وتشمل الطبقات الأخرى الممكنة: طبقة دورة المختنق وطبقة دورة المرشح والمنسوجات الأرضية، ولم يتم استخدامها.

• تطبيق الأسفلت المسامي في شوارع مختارة في مدينة نابلس كبديل للإسفلت التقليدي له فوائد اقتصادية مهمة، حيث يوفر حوالي 1.912.000 شيكل على مدى عمر المشروع وهي 20 سنة. يشمل هذا التوفير تكلفة البناء الأولية (تكاليف المواد وتكاليف نظام تصريف المياه)، وتكاليف الصيانة والتشغيل المستقبلية (تكلفة صيانة الشوارع، وتكلفة صيانة نظام تصريف المياه، وتكلفة الحصول على آلة التنظيف وتشغيلها وصيانتها).

• يوفر استخدام الأسفلت المسامي تكلفة إنشاء وصيانة نظام تصريف للمياه، حيث يسمح لجريان مياه الأمطار بالترشيح من خلال الفراغات. ومع ذلك، يجب تنظيف الإسفلت المسامي بشكل دوري مرتين في السنة للحفاظ على المسامات، وهذا يؤدي إلى زيادة تكلفة الصيانة.

• توصلت نتائج تقييم الأثر البيئي إلى ان إعادة شحن المياه الجوفية يكون بمعدل 107404.7 م 3 / سنة. يمكن أن تلبي هذه الكمية من المياه احتياجات الفرد اليومية في مدينة نابلس لمدة 7 أيام بينما تولد ما يزيد عن 37،023،350 شيكل سنويًا للحكومة.

• استخدام الأسفلت المسامي في نابلس له فوائد بيئية عديدة:

1. تحسين جودة المياه بالفلترة عن طريق المسامات وتقليل الملوثات.

2. تقليل جريان المياه السطحي مما يؤدي إلى تقليل التلوث الحراري مقارنة بالإسفلت التقليدي.

3. المساهمة في حل مشكلة الفيضاناات في مدينة نابلس.

4. المساهمة في تقليل نسبة الحوادث.

5. تؤدي تنقية مياه الأمطار إلى تقليل التكلفة وأعباء العمل المفروضة على محطات تنقية المياه في الشتاء.