# 6 Design of Shear wall

# 6.1 Introduction:

Two types of walls are used in the project, shear wall and masonry walls. Masonry walls considered as non structural element which can be damaged during strong earthquakes.

Shear walls are structural walls in a frame building, they posses necessary stiffness needed to reduce the relative inter-story distributions caused by seismic –induced motion.

Their additional function is to reduce the possibility of damage of nonstructural elements.

Shear walls acts as a cantilever beams. They may be affected by the shear force or by over turning moment under the following condition.

If h/L > 2, the over turning moment is more critical than shear force.

If h/L ≤ 2,

Where:

-h: is the height of shear wall.

-L: the width of shear wall.

# 6.2 Design Limitation on Shear wall:

Shear walls may be divided into two parts "Web and Boundary zones ".Reinforcing of web or none reinforcing is depending on the lateral and axial loads at shear walls .Boundary zones will be treated as columns.

The web of shear walls will be reinforced with ratio (𝜌min=0.25%) in the two direction "vertical and horizontal", and this ratio is for a steel strength 4200 kg/cm2.

# 6.3 Shear strength of the walls:

Shear strength of walls =Vu=фVn

If Vu>Avc√𝒇c, two layers of steel reinforcement is required.

σ=Mc/I

Where:

*σ: the stress at shear wall

*I: moment of inertia of shear wall section.

*c: the diine of neutral axis

*M: the overturning moment.

6.4 Design wall

6.4.A Block A

fig (6-1) shear wall  of Block A

Design of wall1 (W1):

𝒒1=-208 ton /m2     𝒒2=5 ton/m2

L= 4.92m   h=9m.

Fig (6-2) S22 diagram for Sh W1

(𝒒1/c)= (𝒒2/c-L)

(5/4.92-c)= (20.8/c)

c=4.8 m

I= (1/12) (0.3) (4.92)3=2.98 m4

M=q*I/c

=208*2.98/4.8 =129 ton.m

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*129*105/.9*4922*30*240)

=.0005< 𝜌min.

As= 𝜌min*b*h.

=0.0025*30*492

=36.9 cm2

As(19ф16)1ф16@25 cm   c/c

h/L=9/4.92 =1.83<2     shear is critical

Fig (6-3 ) S12 diagram for W1

From SAP:

S12=143.5 ton

Vu=143.5*0.3*4.8=206.64 ton

Vn=Vu/ ф

=206.64/0.75 =275.5 ton

Use two layer of steel because Vu>2Av√ 𝒇c

𝓥c=0.53√ 𝒇c΄* bw*d

Ф Vc=0.75*.035*√240*30*492*10-3

=90.9

Vs=Vn- Ф Vc=275.5- 96.9=184.6 ton

Av/S=Vs/( ℱy*L)

Av/S=0.089

Use Ф10 Av=1.57 cm2

S=1.57/0.089 =17.64 cm

1 Ф10@15 cm c/c

Design of wall2 (W2):

𝒒1=-140 ton /m2     𝒒2=-91 ton/m2

L= 6.36m   h=9m.

Fig ( 6-4) S22diagram for Sh W2

(𝒒1/c)= (𝒒2/c-L)

c=6.36+11.8=18.16 m

I=(1/12)(0.3)(6.36)3=6.43 m4

M=q*I/c

=140*6.43/18.16 =49.57 ton.m

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*49.57*105/0.9*6362*30*240)

=1.07*10-4< 𝜌min.

As= 𝜌min*b*h.

=0.0025*30*636

=47.7 cm2

As(24ф16)1ф16@25 cm c/c

h/L=9/6.36 =1.4<2     shear is critical

Fig (6-5 ) S12 diagram for W2

From SAP:

S12=61.23 ton

Vu=61.23*0.3*18.16=333.6 ton

Vn=Vu/ ф

=333.6/0.75 =444.8 ton

Use two layer of steel because Vu>2Av√ 𝒇c

𝓥c=0.53√ 𝒇c΄* bw*d

Ф Vc=0.75*.035*√240*30*636*10-3

=117.5

Vs =Vn- Ф Vc=444.8- 117.5=327.3 ton

Av/S=Vs/( ℱy*L)

=327.3*103/4200*636

Av/S=0.123

Use Ф10 Av=1.57 cm2

S=1.57/0.123 =12.76 cm

1 Ф10@10 cm c/c

Design of pool wall as Shear wall:

𝒒1=-212 ton /m2     𝒒2=-100 ton/m2

h=3m.

Fig( 6-6)S22diagram for wall pool (𝒒1)

Fig ( 6-7) S22diagram for wall pool (𝒒2)

(𝒒1/c) = (𝒒2/c-L)

c=3+2.68=5.68 m

I= (1/12) (0.25) (3)3=0.5625 m4

M=q*I/c

=212*0.5625/5.68 =21 ton.m

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*21*105/0.9*3002*25*240)

𝜌 < 𝜌min

As= 𝜌min*b*h.

=0.0025*25*300

=18.75 cm2

AS (10ф16)1ф16@30 cm c/c

Fig (6-8 ) S12 diagram for wall pool

From SAP:

S12=83 ton

Vu=83*0.25*3=62.25 ton

Vn=Vu/ ф

=62.25/0.75 =83 ton

𝓥c=0.53√ 𝒇c΄* bw*d

Ф Vc=0.75*.035*√240*30*100*10-3

=18.47

Vs=Vn- Ф Vc=83- 18.47=64.53 ton

Av/S=Vs/( ℱy*L)

=64.53*103/4200*300

Av/S=0.051

Use Ф10 Av=1.57 cm2

S=1.57/0.051 =31 cm

1 Ф10@25 cm c/c

6.4.B Block B:

fig (6-9) shear wall  of Block B

Design of wall 1 (W1):

𝒒1=-200 ton /m2     𝒒2=-26 ton/m2

L= 10.6m   h=8.5m.

Fig ( 6-10) S22diagram for Sh W1

(𝒒1/c)= (𝒒2/c-L)

26/c=200/10.6+c

c=12.2 m

I= (1/12) (0.3) (10.6)3=29.78 m4

M=q*I/c

=200*29.78/12.2 =488 ton.m

ρ=0.85fcfy1-1-2.35Mu∅bd2fc

𝜌=.85*240/4200(1-√ (1-2.35*488*105/0.9*12202*30*240)

=.0004< 𝜌min.

As= 𝜌min*b*h.

=0.0025*30*1060

=79.5 cm2

As(40ф16)1ф16@26 cm c/c

h/L=8.5/10.6 =0.8<2     shear is critical

Fig (6-11 ) S12 diagram for W1

From SAP:

S12=226.4 ton

Vu=226.4*0.3*12.2=333.6 ton

Vn=Vu/ ф

=828.6/0.75 =1105 ton

Use two layer of steel because Vu>2Av√ 𝒇c

𝓥c=0.53√ 𝒇c΄* bw*d

Ф Vc=0.75*.035*√240*30*1060*10-3

=196

Vs=Vn- Ф Vc=1105- 196=909 ton

Av/S=Vs/( ℱy*L)

=909*103/4200*1060

Av/S=0.2

Use Ф10 Av=1.57 cm2

S=1.57/0.2=7.85 cm

1 Ф10@10 cm c/c