Abstract (English)

Cookies Crafter is a fully automated cookie-making system designed to increase efficiency, consistency, and user customizability for small-scale and home production. The project integrates mechanical mechanisms, electronic control, and embedded programming to automate the full production chain: ingredient dispensing and mixing, portion cutting, placement on a rotating tray, optional chocolate topping insertion at user-specified positions, and baking under temperature control via a thermocouple (MAX6675). Ingredient levels are monitored by ultrasonic sensors with LCD alerts for refills; user interaction is provided through a keypad and RFID-based authentication/payment, while an ESP32 module enables remote control via a mobile application. Prototype testing demonstrated reliable dough mixing, accurate cookie positioning, and consistent baking, with identified areas for improvement including sensor calibration, dough-kneading consistency, and scalability for higher throughput. The system offers a practical platform for further enhancements such as additional decoration modules, packaging automation, and predictive maintenance.